#### TABLE OF CONTENTS

CHASSIS

Definitions of School Buses ....................................................................................………2

Table of Capacity and Type Buses ...........................................................................……...3

Air Cleaner  ...........................................................................................................……....4

Axle  ........................................................................................................................……...4

Brakes  ...................................................................................................................……..4-5

Bumper (Front)  .......................................................................................................………5

Certification  ..............................................................................................................…….5

Color  ......................................................................................................................…..…5

Drive Shaft  .............................................................................................................……….5

Electrical System ............................................................................................………..…...6-7

Exhaust System  .................................................................................................…..…7-8

Fenders, Front, Type C Vehicles  ...............................................................................…..8

Frame  ...............................................................................................................……....8

Fuel Tank ..........................................................................................................…………8-9

Governor ...........................................................................................................….……....9

Heating System, Provisions For  ..........................................................................…….…....9

Horn  .................................................................................................................…..…..….9

Instruments and Instrument Panel  ......................................................................……….9-10

Oil Filter  .............................................................................................................………10

Openings ............................................................................................................……...10

Passenger Load  ...................................................................................................……10-11

Power and Grade Ability  ......................................................................................……..11

Shock Absorbers  ..................................................................................................……..11

Steering Gear....................................................................................................…………..11

Suspension Systems..................................................................................………………11-12

Throttle  ...........................................................................................………………………….12

Tires and Rims  .................................................................................................………..12

Transmission ..............................................................................................……..........…..12

Turning Radius  .............................................................................................……....……12

Undercoating  .................................................................................................…….....…13

Wheels  ..........................................................................................................……………..13

BODY

Aisle  ..............................................................................................................…………..…14

Backup Warning Alarm  ....................................................................................…………..14

Battery  ...............................................................................................................………14

Bumper (Front)  ....................................................................................................……...14

Bumper (Rear)  ....................................................................................................…….14-15

Ceiling  .....................................................……………………………………………..………………...15

Certification  ........................................................................................................…...…15

Chains (Tire)  ....................................................................................................………….15

Color  .................................................................................................................………..15

Construction  .........................................................................................................…....15

Crossing Control Arm Magnetic/Stowing Device...……................................…….....…..…….16

Defrosters ............................................................................................................…….16-17

Doors  ..................................................................................................…….......….……17-18

Emergency Equipment  .......................................................................……..........…….18-19

Emergency Exits  ......................................................................................……...…..…..19-20

Floors  ..........................................................................................................………….20-21

Handrails………………………………………………………………………………………………………………..21

Heaters  ..........................................................................................................………..21-22

Hinges ..............................................................................................................……..…..22

Identification  ...................................................................................................………22-23

Inside Height  .......................................................................................................……23

Insulation  .............................................................................................................…..23

Interior  ...........................................................................................................…..…….24

Lamps and Signals  .............................................................................................….…24-26

Metal Treatment  ................................................................................................….....26

Mirrors ..............................................................................................................……….27

Mounting  ...........................................................................................................……...27

Overall Length  .......................................................................................................…..27

Overall Width  ........................................................................................................…...27

Reflective Material  ....................................................................................................27-28

Rub Rails  ..............................................................................................................…….28

Seat Belt for Driver  ...............................................................................................……..28

Seat and Crash Barriers  .......................................................................................…….28-29

Steps  ................................................................................................................….…..29

Step Treads  .......................................................................................................……..30

Stirrup Steps  .......................................................................................................………30

Stop Signal Arm  ..................................................................................................……..30

Strobe Light  .......................................................................................................………30

Sun Shield  .........................................................................................................….…….31

Tailpipe ................................................................................................................…………31

Tow Eyes or Hooks  ..............................................................................................………31

Undercoating  ....................................................................................................………31-32

Ventilation  ...........................................................................................................……….32

Wheelhousing  .....................................................................................................…….32-33

Windows ................................................................................................................…...33

Windshield Washers  ...............................................................................................…..33

Windshield Wipers  ...............................................................................................………33

Wiring  ................................................................................................................…….33-35

SPECIALLY EQUIPPED SCHOOL BUSES

Introduction ..................................................................................................…….………..36

Definition  .....................................................................................................……..……..36

General Requirements  ......................................................................................…….……..37

Aisles ..................................................................................................................……….37

Glazing………………………………………………………………………………………………………………………37

Identification  ......................................................................................................………37

Passenger Capacity Rating  ...................................................................................……….37

Power Lifts and Ramps  ......................................................................................………37-40

Regular Service Entrance  ..................................................................…………..........……...41

Restraining Devices…………………………………………………………………………..…………………….41

Seating Arrangements...........................................................................................…….41

Securement and Restraint System for Wheelchair/Mobility Aid and Occupant …….....…….42-43

Special Light  ........................................................................................................…………43

Special Service Entrance  ...................................................................................……………43

Special Service Entrance Doors  .........................................................................……….…..44

Support Equipment and Accessories  ...................................................................…..……...45

Technology and Equipment, New  .....................................................................…………….46

OPTIONAL EQUIPMENT

Additional Heaters  ............................................................................................…………..47

Air Brakes  ...........................................................................................................………47

Air Conditioner  ...................................................................................................…….47-50

Auxiliary Fan  .....................................................................................................…….….50

Door Lock  ..........................................................................................................………….50

Driver's Seat, Highback Type  ...................................................................................….….50

Extended Warranty  ................................................................................................….….50

Flat Floor  ............................................................................................................…….50-51

Fuel Tank, Sixty Gallon  ......................................................................................……….…51

Heat Booster Pump ..........................................................................……… ..................51

Power Lift and Ramp  ..........................................................................................………….51

Public Address System……………………………………………………………………………………….………51

Reflective Material……………………………………………………………………………………………………51

Restraining Devices.............................................................................................………..51

Stop Arm, Air Operated……….……….................................................................…….…………51

Storage Box  .........................................................…...........................................………..…51

Storage Compartment (Luggage)  ..................................................................………………52

Tinted Glazing  .............................................................................................…………………52

White Roof  ................................................................................................……...………….52

SERVICE VEHICLES

Service Vehicle Standards  .......................................................................……..........………53

ALTERNATIVE FUELS

Alternative Fuel Standards  ....................................................................……....………….53-63

GENERAL PROVISIONS

The Minimum Standards contained herein apply to all school transportation vehicles purchased by school districts of Mississippi.  No school district has authority under Mississippi laws and Mississippi Board of Education regulations to purchase or use any equipment except the type approved by the Mississippi Board of Education.

Federal regulations require that all vehicles manufactured and sold for the purpose of transporting school children meet applicable Federal Motor Vehicle Safety Standards.  It is the responsibility of the manufacturer and seller to make sure that all school buses are certified according to the standards.

Each vehicle shall be properly serviced prior to being delivered to the school district.

Any reference to "or equal" means that manufacturers of the chassis or body (or complete bus) for which a bid is submitted shall have the authority to determine equal quality of parts and components to be considered.

A representative of the Division of Pupil Transportation, Mississippi Department of Education, Jackson, Mississippi, shall have the privilege of visiting school bus manufacturing plants of successful bidders for the purpose of inspecting school bus chassis, school bus bodies, or combinations thereof, both during construction and after completion for delivery to Mississippi school districts.

Reference to SBMTC in the Standards is the School Bus Manufacturers Technical Committee and reference to SAE in the Standards is the Society of Automotive Engineers.

DEFINITIONS OF SCHOOL BUSES

###### TYPE A

A Type A school bus is a conversion bus constructed utilizing a cutaway front section vehicle with a left side driver’s door. This definition includes two classifications: Type A-1, with a Gross Vehicle Weight Rating (GVWR) of 14,500 pounds or less; and Type A-2, with a GVWR greater than 14,500 pounds and less than or equal to 21,500 pounds.

TYPE B

A Type B school bus is constructed utilizing a stripped chassis. The entrance door is behind the front wheels. This definition includes two classifications: Type B-1, with a GVWR of 10,000 pounds or less, and Type B-2, with a GVWR greater than 10,000 pounds.

TYPE C

A Type C school bus is constructed utilizing a chassis with a hood and front fender assembly. The entrance door is behind the front wheels-also known as a conventional style school bus. This type also includes the cutaway truck chassis or truck chassis with cab with or without a left side door and with a GVWR greater than 21,500 pounds.

TYPE D

A Type D school bus is constructed utilizing a stripped chassis. The entrance door is ahead of the front wheels-also known as a rear engine or front engine transit style school bus.

MINIMUM STANDARDS FOR SCHOOL BUSES

TABLE OF CAPACITY AND TYPE

| TYPE(S) | CAPACITY |
| --- | --- |
| Type A-I Converted Van Cutaway Van | 16 – 17, 18 - 20 |
| Type A-II Cutaway Van (Dual Wheels) | 16 – 18, 20 - 23 |
| Type A-II Cutaway Van (Dual Wheels) | 24, 30 |
| Type B | 25-28, 29-33, 35-38, 63, 69, 75 |
| Type C - Conventional Flat Face Cowl | 29, 35, 41, 47, 53, 59, 65, 71, 77 |
| Type D - Transit Mid-range Front & Rear Mounted Engine | 41, 47, 53, 59, 65, 71, 77, 84 |
| Type D - Transit Front & Rear Mounted Engine | 71, 77, 83, 89 |

# AIR CLEANER

1.	The engine intake air cleaner (system) shall be furnished and properly installed by the chassis manufacturer to meet engine (manufacturer's) specifications.

2.	The intake air system for diesel engines may have an air cleaner restriction indicator properly installed by the chassis manufacturer to meet engine specifications.

AXLE

The front and rear axle and suspension system shall have a gross axle weight rating (GAWR), at ground commensurate with the respective front and rear weight loads that will be imposed by the bus.

BRAKES

1.	The braking system shall include the service brake, an emergency brake that is a part of the service brake system and controlled by the service brake control, and a parking brake.

2.	Buses using air in the operation of the brake system shall be equipped with warning signals, readily audible and visible to the driver, that will give a continuous warning when the air pressure available in the system for braking is sixty (60) psi (pounds per square inch).

An illuminated gauge shall be provided that will indicate to the driver the air pressure in pounds per square inch available for the operation of the brakes.

Any brake system with a dry reservoir shall be equipped with a check valve or

equivalent device to ensure that in the event of failure or leakage in its connection

to the source of compressed air, the stored dry air shall not be depleted by the

leakage or failure.

3.	Buses using a hydraulic assist-brake shall be equipped with warning signals, readily audible and visible to the driver, that will provide continuous warning in the  event of a loss of fluid flow from primary source and in the event of discontinuity in that portion of the vehicle electrical system that supplies power to the back-up system.

4.	The brake lines and booster-assist lines shall be protected from excessive heat and vibrations and installed in a manner which prevents chafing.

5.	All brake systems shall be designed to permit visual inspection of brake lining wear without removal of any chassis components.

6.	Type D buses (71 capacity and above) shall be equipped with air brakes.

7.	Antilock brake systems for either air or hydraulic brakes shall include control of all axles in compliance with Federal Motor Vehicle Safety Standards (FMVSS) 105 or 121.

BUMPER (FRONT)

1.	All school buses shall be equipped with a front bumper.  The front bumper shall be furnished by the chassis manufacturer as part of the chassis on all types of chassis unless there is a specific arrangement between the chassis manufacturer and body manufacturer that the body manufacturer will furnish the front bumper.

2.	The front bumper shall be of pressed steel channel or equivalent material at least three-sixteenths  (3/16) of an inch  thick and not less than eight inches (8") wide (high) and shall extend beyond the forward-most part of the body, grille, hood and fenders and shall extend to the outer edges of the fenders at the bumper's top line.

3.	The front bumper, except breakaway bumper ends, shall be sufficient strength to permit pushing a vehicle of equal gross vehicle weight without permanent distortion to the bumper, chassis or body.

4.	Tow eyes or hooks shall be furnished on Type B over 35 passenger or C and D buses and attached so as not to project beyond the front bumper.  Tow eyes or hooks attached to the frame chassis shall be furnished by the chassis manufacturer. This installation shall be in accordance with the chassis manufacturer's standards.

5.	The bumper shall be designed or reinforced so that it will not deform when the bus is lifted by a chain that is passed under the bumper (or through the bumper if holes are provided for this purpose) and attached to both tow eyes.  For the purpose of   meeting this standard, the bus shall be empty and positioned on a level, hard         surface and both tow eyes shall share the load equally.

# CERTIFICATION

Chassis manufacturers will, upon request, certify to the Mississippi Department of Education, that their product meets state minimum standards on items not covered by certification issued under requirements of the National Traffic and Motor Vehicle Safety Act.

COLOR

The chassis, including wheels and front bumper, shall be black.  The hood, cowl and fenders shall be in national school bus yellow.

DRIVE SHAFT

The drive shaft shall be protected by a metal guard or guards around the circumference of the drive shaft to reduce the possibility of its whipping through the floor or dropping to the ground if broken.

ELECTRICAL SYSTEM

1.	Battery

- Storage battery shall have minimum cold cranking capacity rating equal to the cranking current required for 30 seconds at 0 degrees Fahrenheit (-17.8 C) and a minimum reserve capacity rating of 120 minutes at 25 amps.  Higher capacities may be required depending upon optional equipment and local environmental conditions.

- Since all batteries are to be secured in a sliding tray in the body, chassis manufacturers shall temporarily mount the battery on the chassis frame, except that van conversion or cutaway front-section chassis may be manufacturer’s standard configuration.  In all cases, however, the battery cable provided with the chassis shall have sufficient length to allow some slack.

2.	Alternator

a.	All Type A buses and Type B buses up to fifteen thousand pounds (15,000 lbs.) GVWR shall have a minimum one hundred thirty (130) ampere alternator.

b.	All buses over 15,000 lbs. GVWR  shall be equipped with a heavy-duty truck or bus-type alternator meeting SAE J 180, having a minimum output rating of one hundred seventy- five (175) amperes.

c.	Direct-drive alternator is permissible in lieu of belt drive.  Belt drive shall be capable of handling the rated capacity of the alternator with no detrimental effect on other driven components.

For estimating required alternator capacity, see School Bus Manufactures Technical Council’s publication, “School Bus Technical Reference.”

3.	Wiring

a.	All wiring shall conform to current applicable recommended practices of the Society of Automotive Engineers (SAE).

(1)	All wiring shall use a standard color and number coding and each chassis shall be delivered with a wiring diagram that coincides with the wiring of the chassis.

- Chassis manufacturer shall install a readily accessible terminal strip or plug (Type A manufacturer's standard) on the body side of the cowl, or at an accessible location in the engine compartment of vehicles designed without a cowl, that shall contain the following terminals for the body connections:

(1.) Main one hundred (100) amp body circuit.

(2.) Tail lamps

(3.) Right turn signal

(4.) Left turn signal

(5.) Stop lamps

(6.) Back-up lamps

(7.) Instrument panel light (rheostat controlled by head lamp switch).

4.	Circuits

a.	An appropriate identifying diagram (color and number coded) for electrical circuits shall be provided to the body manufacturer for distribution to the end user.  Type A and B shall be manufacturer's standard.

b.	Headlight system must be wired separately from the body-controlled solenoid.

EXHAUST SYSTEM

1.	Exhaust pipe, muffler and tailpipe shall be outside the bus body compartment and attached to the chassis so as not to damage any other chassis component.

2.	The tailpipe shall be constructed of a corrosion-resistant tubing material at least equal in strength and durability to 16-gauge steel tubing.

3.	Chassis manufacturers shall furnish an exhaust system with tailpipe of sufficient length to exit the rear of the bus or at the left side of the bus body no more than eighteen inches (18") forward of the front edge of the rear wheel house opening. If designed to exit at the rear of the bus, the tailpipe shall extend at least five inches beyond the end of the chassis frame.  If designed to exit to the side of the bus, the tailpipe shall extend at least 48.5" inches (51.5 inches if the body is to be 102 inches wide) outboard from the chassis centerline.

a.	On Types C and D vehicles, the tailpipe shall not exit beneath a fuel fill or emergency door exit.

b.	Type A and B chassis may be furnished with the manufacturer’s standard tailpipe configuration.

4.	Exhaust system on a chassis shall be adequately insulated from the fuel system.

5.	Muffler shall be constructed of corrosion-resistant material.

6.	The exhaust system on vehicles equipped with a power lift unit may be routed to the left of the right frame rail to allow for the installation of a power lift unit on the right side of the vehicle.

FENDERS, FRONT, TYPE C VEHICLES

1.	The total spread of the outer edges of the front fenders, measured at the fender line, shall exceed the total spread of the front tires when the front wheels are in a straight-ahead position.

2.	The front fenders shall be properly braced and free from any body attachments.

FRAME

1.	The frame or equivalent shall be of such design and strength characteristics as to correspond at least to the standard practice for trucks of the same general load characteristics which are used for highway service.

2.	Any secondary manufacturer that modifies the original chassis frame shall guarantee the performance of workmanship and materials resulting from such modification.

3.	Frames shall not be modified for the purpose of extending the wheel base.

4.	Holes in the top or bottom flanges or side units of the frame, and welding to the frame, shall not be permitted except as provided or accepted by the chassis manufacturer.

FUEL TANK

1.	Fuel tank or tanks having a thirty gallon (30 gal.) capacity with a twenty-five  gallon          (25 gal.) actual draw shall be provided by the chassis manufacturer for Types A, B,            C and D mid-range (fifty-three [53] capacity or less) school buses.  Types B, C and            D mid-range (greater than fifty-three (53) capacity) and Type D transit buses shall             be equipped with a fuel tank of minimum sixty gallon (60 gal.) capacity with an

actual draw capacity of eighty-three per cent (83%).  The tank shall be filled and     vented to the outside of the body, the location of which shall be so that accidental   fuel spillage will not drip or drain on any part of the exhaust system.

2.	No portion of the fuel system which is located outside of the engine compartment, except the filler tube, shall extend above the top of the chassis frame rail.  Fuel lines shall be mounted to obtain maximum possible protection from the chassis frame.

3.	A fuel filter with replacement element shall be installed between the fuel tank and the engine.

4.	Installation of alternative fuel systems, including fuel tanks and piping from tank to engine, shall comply with all applicable fire codes and applicable Federal Motor Vehicle Safety Standards in effect on the date of manufacture of the bus.

Installation of LPG tanks shall comply with National Fire Protection Association

(NFPA) 58.

5.	The fuel tank on vehicles constructed with a power lift unit may be mounted on the left chassis rail or behind the rear wheels.

GOVERNOR

###### An electronic engine speed limiter shall be provided and set to limit engine speed, not to exceed the maximum revolutions per minute, as recommended by the engine manufacturer.

HEATING SYSTEM, PROVISIONS FOR

The chassis engine shall have plugged openings for the purpose of supplying hot water for the bus heating system.  The opening shall be suitable for attaching three-fourths  inch (3/4") pipe thread/hose connector.  The engine shall be capable of supplying water having a temperature of at least one hundred and seventy Fahrenheit  (170) at a flow rate of fifty  pounds (50 lbs.) /per minute at the return end of thirty  feet  (30 ft.) of one inch (1") inside diameter automotive hot water heater hose.  (SBMTC Standard No. 001-Standard Code for Testing and Rating Automotive Bus Hot Water Heating and Ventilating Equipment).

HORN

The bus shall be equipped with horn or horns of standard make with each horn capable of producing a complex sound in bands of audio frequencies between two hundred and fifty (250) and two thousand (2,000) cycles per second and tested in accordance with SAE  J-377.

INSTRUMENTS AND INSTRUMENT PANEL

1.	The chassis shall be equipped with the following instruments and gauges.  (Lights in lieu of gauges are not acceptable except as noted):

a.	Speedometer

b.	Odometer which will give accrued mileage (to seven (7) digits) including tenths of miles.

c.	Voltmeter

d.	Oil pressure gauge

e.	Water temperature gauge

f.	Fuel gauge

g.	Upper beam headlight indicator

- Brake indicator gauge (vacuum or air) - Light indicator in lieu of gauge

permitted on vehicle equipped with hydraulic-over-hydraulic brake system.

i.	Turn signal indicator

- Glow-plug indicator light where appropriate.

2.	All instruments shall be easily accessible for maintenance and repair.

3.	Instruments and gauges shall be mounted on the instrument panel in such a manner that each is clearly visible to the driver while seated in a normal driving position.

4.	The instrument panel shall have lamps of sufficient candlepower to illuminate all of the instruments and gauges and shift selector indicator for automatic transmission.

# OIL FILTER

An oil filter with a replaceable element shall be provided and connected by flexible oil lines if it is not a built-in or an engine-mounted design. The oil filter shall have a capacity in accordance with the engine manufacturer’s recommendation.

OPENINGS

All openings in the floorboard or firewall between the chassis and passenger compartment, such as for the gearshift selector and parking brake lever, shall be sealed.

PASSENGER LOAD

1.	The actual gross vehicle weight (GVW) is the sum of the chassis weight, plus the body weight, plus the driver's weight, plus the total seated pupil weight.

a.	For purposes of calculation, the driver's weight is 150 pounds.

b.	For purposes of calculation, the pupil weight is 120 pounds per pupil.

2.	Actual gross vehicle weight (GVW) shall not exceed the chassis manufacturer's gross vehicle weight rating (GVWR) for the chassis nor shall the actual weight carried on any axle exceed the chassis manufacturer’s GVWR.

3.	Manufacturer's (GVWR) shall be furnished in duplicate (unless more are requested)  by manufacturers to the Mississippi Department of Education.  The Mississippi        Department of Education shall, in turn, transmit such ratings to other state agencies responsible for development or enforcement of state standards for school buses.

POWER AND GRADE ABILITY

Gross Vehicle Weight Rating (GVWR) shall not exceed 185 pounds per published net horsepower of the engine at the manufacturer’s recommended maximum number of revolutions per minute.

SHOCK ABSORBERS

The bus shall be equipped with front and rear double-action shock absorbers compatible with the manufacturers rated axle capacity at each wheel location.

STEERING GEAR

1.	The steering gear shall be approved by the chassis manufacturer and designed to ensure safe and accurate performance when the vehicle is operated with a maximum load and at maximum speed.

2.	If external adjustments are required, the steering mechanism shall be accessible to accomplish the same.

3.	No changes shall be made in the steering apparatus which are not approved by the chassis manufacturer.

4.	There shall be a clearance of at least two inches (2") between the steering wheel and cowl, instrument panel, windshield or any other surface.

5.	Power steering is required and shall be of the integral type with integral valves.

6.	The steering system shall be designed to provide means for lubrication of all wear-

points, if wear-points are not permanently lubricated.

SUSPENSION SYSTEM

1.	The capacity of springs or suspension assemblies shall be commensurate with the chassis manufacturer’s GVWR.

2.	Rear leaf springs shall be of a progressive rate or multi-stage design. Front leaf springs shall have a stationary eye at one end and shall be protected by a wrapped leaf, in addition to the main leaf.

THROTTLE

The force required to operate the throttle shall not exceed sixteen (16) pounds throughout the full range of accelerator pedal travel.

TIRES AND RIMS

1.	Tires and rims of proper size and tires with a load rating commensurate with the      chassis manufacturer's gross vehicle weight rating shall be provided. The use of multi-piece rims and\or tube-type tires shall not be permitted.

2.	Dual rear tires shall be provided on Type A-1, Type B, Type C and Type D school buses.

3.	All tires on any given vehicle shall be of the same size and the load range of said tires shall meet or exceed the gross axle weight rating as required by FMVSS -120.

- If the vehicle is equipped with a spare tire and rim assembly, it shall be of the same size as those mounted on the vehicle.

5.	If a tire carrier is required, it shall be suitably mounted in an accessible location outside of the passenger compartment.

6.	Tubeless radial tires shall be required for all school buses.

TRANSMISSION

Automatic transmissions shall be standard on Types A, B, C and D school buses and shall have no fewer than three forward speeds and one reverse speed. Mechanical shift selector shall provide a detent between each gear position when the gear selector quadrant and shift selectors are not steering column mounted.

TURNING RADIUS

1.	Chassis with a wheelbase of two hundred and sixty-four inches (264") or less shall have a right and left turning radius of not more than forty-two and one half feet (42 ½’), curb to curb measurement.

2.	Chassis with a wheelbase of two hundred and sixty-five inches (265") or more shall

have a right and left turning radius of not more than forty-four and one half feet  (44½’), curb to curb measurement.

UNDERCOATING

The chassis manufacturer, or agent thereof, shall coat the undersides of steel or metallic-constructed front fenders with rust-proofing compound for which compound manufacturers have issued notarized certification of compliance to chassis builder that compound meets or exceeds all performance and qualitative requirements of paragraph 3.4 of Federal Specification TT-C-520B using modified test.

WHEELS

The bus shall be equipped with disc wheels.

##### MINIMUM STANDARDS FOR SCHOOL BUS BODIES

AISLE

1.	All emergency doors shall be accessible by a twelve inch (12") minimum aisle.  Aisle shall be unobstructed at all times by any type of barrier, seat, wheelchair or tiedown, unless a flip seat is installed and occupied.  A flip seat in the unoccupied (up) position shall not obstruct the twelve inch (12") minimum aisle to any side emergency door.

2.	The seat backs shall be slanted sufficiently to give aisle clearance of fifteen inches  (15") at tops of seat backs.

BACK-UP WARNING ALARM

An automatic audible alarm shall be installed behind the rear axle and shall comply with the Society of Automotive Engineers published Back-up Alarm Standards (SAE J994b) a minimum of one hundred seven (107) dba for rubber tired vehicles.

BATTERY

1.	A battery is to be furnished by the chassis manufacturer.

2.	When the battery is mounted as described in the chassis section, the body manufacturer shall securely attach the battery on a slide-out or swing-out tray in a closed, vented compartment in the body skirt, so that the battery is accessible from the outside for convenient servicing.  The battery compartment door or cover shall be hinged at the front or top and secured by an adequate and conveniently operated latch or other type of fastener.  On all Type A buses, one or both batteries may be mounted in the engine compartment in an accessible location.

BUMPER (FRONT)

On a Type D school bus, if the chassis manufacturer does not provide the bumper, it shall be provided by the body manufacturer.  The bumper will conform to standards in the chassis section.

BUMPER (REAR)

- The bumper shall be of pressed steel channel or equivalent material at least three sixteenths (3/16") of an inch thick and shall be a minimum of eight inches (8") wide (high) on Type A-1, and shall be minimum of nine and one-half inches (9 1/2") wide (high) on Types A-2, B, C, and D buses and of sufficient strength to permit pushing by another vehicle without permanent distortion.

2.	Bumper shall be wrapped around the back corners of the bus.  It shall extend forward at least twelve inches (12"), measured from rear-most point of the body at the floor line and shall be flush mounted to body side or protected with an end panel.

3.	Bumper shall be attached to the chassis frame in such a manner that it may be easily removed.  It shall be so braced as to withstand impact from a rear or side impact.  It shall be so attached as to discourage hitching of rides.

4.	Bumper shall extend at least one inch (1") beyond the rear-most part of the body surface measured at the floor line.

5.	The bumper by the chassis manufacturer may be used on Type A conversion vehicles.

CEILING

See Insulation and Interior, Body section.

CERTIFICATION

The body manufacturer will, upon request, certify to the Mississippi Department of Education that their product meets state minimum standards on items not covered by certification issued under requirements of the National Traffic and Motor Vehicle Safety Act.

CHAINS (TIRE)

See Wheelhousing, Body section.

COLOR

1.	The school bus body shall be painted national school bus yellow.

- The body exterior paint trim, rub rails, bumper, lamp hoods, emergency door  arrow, and lettering shall be black.

CONSTRUCTION

- Construction shall be of prime commercial quality steel or other metal or material with strength at least equivalent to all-steel as certified by the bus body manufacturer.

- Construction shall be reasonably dust proof and watertight.

CROSSING CONTROL ARM MAGNETIC/STOWING DEVICE

1.	Buses shall be equipped with a crossing control arm mounted on the right side of the front bumper, which shall not open more than ninety degrees (90).

2.	All components of the crossing control arm and all connections shall be weatherproofed.

3.	The crossing control arm shall incorporate system connectors (electrical, vacuum, or air) at the gate and shall be easily removable to allow for towing of the bus.

4.	The crossing control arm shall meet or exceed SAE Standard J1133.

5.	The crossing control arm shall be constructed of non-corrosive or nonferrous material or treated in accordance with the body sheet metal standard (see METAL TREATMENT).

6.	There shall be no sharp edges or projections that could cause hazard or injury to students.

7.	The crossing control arm shall extend a minimum of 70 inches (measured from the bumper at the arm assembly attachment point) when in the extended position. The crossing control arm shall not extend past the end of the bumper when in the stowed position.

8.	The crossing control arms shall extend simultaneously with the stop arm(s) by                   means of the stop arm controls.

- An Automatic recycling interrupt switch shall be installed for temporary disabling of

the crossing control arm.

10.	The crossing control arm shall be equipped with an electromagnetic device or a similar assembly attached to the bumper at the end of the arm to provide a positive latch when in the closed mode.

DEFROSTERS

1.	Defrosting and defogging equipment shall direct a sufficient flow of heated air onto the windshield, the window to the left of the driver and the glass in the viewing area directly to the right of the driver to eliminate frost, fog and snow.

2.	The defrosting system shall conform to the Society of Automotive Engineers Performance Standards J-381 and 382.

3.	The defroster and defogging system shall be capable of furnishing heated, outside, ambient air except that part of the system furnishing additional air to the windshield, entrance door and stepwell may be of the re-circulating air type.

4.	Auxiliary fans are not considered defrosting or defogging systems.

5.	Portable heaters may not be used.

DOORS

Service Door

a.	Service door shall be in the driver’s control, and designed to afford easy release and provide a positive latching device on manual operating doors  to prevent accidental openings.  When a hand lever is used, no part shall come together that will shear or crush fingers.  Manual door controls shall not require more than twenty-five (25) pounds of force to operate at any point throughout the range of operation on level surface.

b.	The service door shall be located on the right side of the bus, opposite the driver and within direct view of the driver.

- The service door shall have a minimum horizontal opening of twenty-four      inches (24") and a minimum vertical opening of sixty-eight inches (68").  Type A-1 vehicles shall have a minimum opening area of twelve hundred

(1,200) square inches).

d.	The service door shall be of a split-type, sedan type or jack-knife type.  (Split-type door includes any sectioned door which divides and opens inward or outward).  If one section of a split-type door opens inward and the other opens outward, the front section shall open outward.

e.	The lower as well as the upper panels shall be of approved safety glass.  The bottom of each lower glass panel shall not be more than ten inches  (10") from the top surface of the bottom step.  The top of each upper glass panel shall not be more than three inches (3") from the top of the door.  Type A vehicles shall have upper panel (windows) of safety glass with an area of at least three hundred fifty (350) square inches.

f.	Vertical closing edges on split-type or folding type entrance doors shall be equipped with flexible material to protect the children's fingers. Type A-1 vehicles may be equipped with the chassis manufacturer's standard entrance door.

g.	There shall be no service door to the left of the driver on Type B, C or D vehicles.  Type A vehicles may be equipped with the chassis manufacturer's standard door.

h.	All doors shall be equipped with padding at the top edge of each door opening.  The pad shall be at least three inches (3") wide and one inch (1") thick and extend the full width of the door opening.

# EMERGENCY EQUIPMENT

1.	Fire Extinguisher

a.	The bus shall be equipped with at least one (1) pressurized, dry, chemical fire extinguisher, complete with hose, to meet Underwriters Laboratories, Inc., approval.  The extinguisher shall be mounted in a bracket, located in the driver's compartment and readily accessible to the driver and passengers.  A pressure gauge shall be mounted on the extinguisher so as to be easily read without moving the extinguisher from its mounted position.  The fire extinguisher shall not be located in the entrance door area.

- The fire extinguisher shall be of a type approved by Underwriter                        Laboratories, Inc., with a total rating of 2A10BC or greater. The operating          mechanism shall be sealed with a type of seal which will not interfere with         the use of the fire extinguisher.

2.	First-Aid Kit

a.	The bus shall have a removable, moisture-proof and dust-proof first-aid kit in an accessible place in the driver's compartment.  It shall be properly mounted and identified as a first aid kit.  The location for the first aid kit shall be marked.

b.	Each kit shall include:

2 - 1" x 2 1/2 yards adhesive tape rolls

24 - sterile gauze pads 3" x 3"

100 - 3/4" x 3" adhesive bandages

8 - 2" bandage compress

10 - 3" bandage compress

2 - 2" x 6' sterile gauze roller bandages

2 - non-sterile triangular bandages approximately

40" x 36" x 54" with 2 safety pins

3 - sterile gauze pads 36" x 36"

3 - sterile eye pads

1 - round end scissors

1 - pair latex gloves

1 - mouth-to-mouth airway

3.	Body Fluid Clean-up Kit

a.	Each bus shall have a removable and moisture proof body fluid clean-up kit accessible to the driver.  It shall be properly mounted and identified as a body fluid clean-up kit.

b.	Each kit shall include:

1 - 2 oz. package infectious liquid spill control powder

1 - odor reducing mask

2 - latex gloves

2 - antiseptic wipes

2 - paper crepe towels

1 - scraper

1 - plastic disposal bag with scoop and tie

4.	Warning Devices

Each school bus shall contain at least three (3) reflectorized, triangle road warning devices mounted in an accessible place.  These devices must meet requirements FMVSS 125 and shall not be located in the entrance door area.

EMERGENCY EXITS

- Emergency door(s) and other emergency exits shall comply with the requirements

of FMVSS -217 and any of the requirements of these standards that exceed FMVSS-217.

2.	Emergency door requirements

a.	Upper portion of the emergency door shall be equipped with approved safety glazing, exposed area of which shall be at least four hundred  (400) square inches.  The lower portion of the rear emergency doors on Types A-2, B, C and D vehicles shall be equipped with a minimum of three hundred fifty (350) square inches of approved safety glazing.

b.	There shall be no steps leading to an emergency door.

c.	The words “EMERGENCY DOOR,” in letters at least two inches (2") high, shall be placed at the top of or directly above the emergency door, or on the door in the metal panel above the top glass, both inside and outside the bus.

d.	The emergency door(s) shall be equipped with padding at top edge of each door opening.  Padding shall be at least three inches (3") wide and one inch (1") thick, and extend the full width of the door opening.

e.	There shall be no obstruction higher than one-fourth inch (1/4") across the bottom of any emergency door opening.

3.	Emergency exit requirements

- Types A, B, C and D vehicles shall be equipped with a total number of emergency exits as follows for the indicated capacities of vehicles.  Exits required by FMVSS 217 may be included to comprise the total number of exits specified.

| SEATING CAPACITY | ADDITIONAL EXITS REQUIRED |
| --- | --- |
| 0 - 42 | 1 Exit window, per side and 1 roof hatch |
| 43 - 78 | 2 Exit windows, per side and 2 roof hatches |
| 79 - 90 | 3 Exit windows, per side and 2 roof hatches |

b.	Each emergency exit above shall comply with FMVSS 217.  These emergency exits are in addition to the rear emergency door or exit.

c.	In addition to the audible warning required on emergency doors by FMVSS 217, additional emergency exits may also be equipped with an audible warning device.

d.	Simple release mechanism shall be provided on roof exits, permitting operation as emergency exit(s), assessable inside and outside the vehicle.  Roof exits shall be installed, hinged toward the front.

e.	Roof exits shall provide a “partially open” position along the full width of the hatch, adequate to allow air and thereby ventilate the bus.

f.	In addition, all metallic hardware shall be manufactured from corrosion-resistant materials such as aluminum, brass or stainless steel, etc. as to prevent corrosion and possible failure of the opening mechanism.

FLOORS

1.	The floor in the under-seat area, including the tops of the wheelhousing, driver's compartment, and toeboard, shall be covered with rubber floor covering or the equivalent, having a minimum overall thickness of point one twenty-five of an inch (.125").  The driver’s area on all Type A buses may be manufacturer’s standard flooring and floor covering.

2.	The floor covering in the aisle shall be of an aisle-type rubber or equivalent, wear-resistant and ribbed.  The minimum overall thickness shall be point one eighty-sevenths of an inch (.187") measured from the tops of the ribs.

- The floor covering must be permanently bonded to the floor and must not crack when subjected to sudden changes in temperature.  Bonding or adhesive material

shall be waterproof and shall be of a type recommended by the manufacturer of the floor-covering material.  All seams must be sealed with a waterproof sealer.

4.	On Types  B, C and D buses a screw-down plate that is secured and insulated shall be provided to access the fuel tank sending unit.

# HANDRAILS

At least 1 handrail shall be installed. The handrail(s) shall assist passengers during entry or exit, and shall be designed to prevent entanglement, as evidenced by the passing of the NHTSA string and nut test.

HEATERS

1.	Heaters shall be of a hot-water type and/or combustion type.

2.	If only one (1) heater is used, it shall be of a fresh-air or combination fresh-air and re-circulating type.

3.	If more than one (1) heater is used, additional heaters may be of the re-circulating           air type.

4.	The heating system shall be capable of maintaining  bus interior temperatures as specified in SAE test procedure J2233.

5.	Auxiliary fuel-fired heating systems are permitted, provided they comply with the following:

a.	The auxiliary heating system fuel shall utilize the same type of fuel as specified for the vehicle engine.

b.	Heater(s) may be direct hot air or connected to the engine’s coolant system.

c.	Auxiliary heating system, when connected to the engine’s coolant system, may be used to preheat the engine coolant or preheat and add supplementary heat to the bus’s heating system.

d.	Auxiliary heating systems must be installed pursuant to the manufacturer’s recommendations and shall not direct exhaust in such a manner that will endanger bus passengers.

e.	Auxiliary heating systems which operate on diesel fuel shall be capable of operating on #1, #2 or blended diesel fuel without the need for system adjustment.

f.	The auxiliary heating system shall be low voltage.

g.	Auxiliary heating systems shall comply with all applicable Federal Motor Vehicle Safety Standards, including FMVSS 301, as well as, SAE test procedures.

6.	All heaters installed by body manufacturers shall bear a name plate that indicates the heater rating in accordance with SBMTC Standard No. 001.  The plate shall be affixed by the heater manufacturer and shall constitute certification that the heater performance is as shown on the plate.

7.	Heater hoses shall be adequately supported to guard against excessive wear due to vibration.  The hoses shall not dangle or rub against the chassis or sharp edges and shall not interfere with or restrict the operation of any engine function.  The heater hose shall conform to SAE Standard J20c.  The heater lines on the interior of the bus shall be shielded to prevent scalding of the driver or passengers.

8.	Each hot water system installed by a body manufacturer shall include one (1) shut off valve in the pressure line and one (1) shut off valve in the return line with both valves at the engine in an accessible location, except that on all Types A and B buses, the valves may be installed in another accessible location.

9.	There shall be a water flow regulating valve installed in the pressure line for convenient operation by the driver while seated.

10.	All combustion heaters shall be in compliance with current Federal Motor Carrier Safety Regulations.

11.	Accessible bleeder valves shall be installed in an appropriate place in the return lines of body company-installed heaters to remove air from the heater lines.

12.	Access panels shall be provided to make heater motors, cores and fans readily accessible for service.  Outside access panel may be provided for the driver's heater.

HINGES

All exposed metal door hinges subject to corrosion shall be designed to allow lubrication to be channeled to the center seventy-five percent (75%) of each hinge loop.

IDENTIFICATION

1.	Body shall bear words "SCHOOL BUS" in black letters at least eight inches (8") high on both front and rear of the body or on signs attached thereto.  Lettering shall be placed as high as possible without impairment of its visibility.  Letters shall conform to "Series B" of the Standard Alphabets for highway signs.

2.	Required lettering and numbering shall include:

a.	District or company name or owner of the bus shall be displayed at the beltline in black, unshaded letters, not less than five inches (5") high.

b.	Bus identification number shall be displayed on the sides, rear and on the front in black or contrasting unshaded numbers not less than five inches  (5") high.

3.	Other lettering, numbering, or symbols which may be displayed on the exterior of the bus, shall be limited to:

a.	Bus identification number on the top of the bus, in addition to required numbering on sides, rear, and front.

b.	The location of the battery(ies) identified by the word “BATTERY” or “BATTERIES” on the battery compartment door in two inch (2") lettering.

c.	Symbols or letters near the service door displaying information for identification by the students of the bus or route served.  Such symbols or lettering, if used, shall not exceed thirty-six inches (36) square in size.

d.	Manufacturer, dealer or school identification.

e.	Symbols identifying the bus as equipped for or transporting students with special needs (see Specially Equipped School Bus section)

f.	Lettering or signs on the rear of the bus relating to school bus flashing signal lamps or railroad stop procedures may be of reflective material.

# INSIDE HEIGHT

The inside body height shall be seventy-two inches (72") or more, measured metal to metal, at any point on longitudinal centerline from the front vertical bow to the rear vertical bow.  The inside body height of Type A-1 buses shall be sixty-two inches (62") or more.

INSULATION

1.	Thermal insulation for ceiling and walls shall be fire-resistant, Underwriters' Laboratories, Inc., approved, and approximately one and one-half inch (1 1/2") thick with minimum R-value of 5.5.  Insulation shall be installed to prevent sagging.

2.	If floor insulation is required, it shall be either five (5) ply, nominal five eighths inch (5/8") thick plywood or material of equal or greater strength and insulation R value, and it will equal or exceed properties of exterior-type softwood plywood, C-D Grade as specified in standard issued by the U. S. Department of Commerce. When plywood is used, all exposed edges shall be sealed.  Type A buses may be equipped with nominal one-half inch (1/2") thick plywood meeting above requirements.

INTERIOR

1.	Interior of bus shall be free of all unnecessary projections, which include luggage racks and attendant handrails, to minimize the potential for injury.  This standard requires inner lining on ceilings and walls.  If ceiling is constructed to contain lapped joints, forward panel shall be lapped by rear panel and exposed edges shall be beaded, hemmed, flanged or otherwise treated to minimize sharp edges.

2.	Interior overhead storage compartments may be provided if they meet the following criteria:

a.	Meet head protection requirements of FMVSS 222, where applicable.

b.	Have a minimum rated capacity displayed for each compartment.

c.	Be completely enclosed and equipped with latching doors.  Doors and latches must be sufficient to withstand a force of five (5) times the maximum rated capacity of the compartment.

- Have all corners and edges rounded with a minimum radius of one inch (1")

or padded equivalent to door header padding.

e.	Must be attached to the bus sufficiently to withstand a force equal to twenty (20) times the maximum rated capacity.

f.	Shall have no protrusions greater than one-fourth inch (1/4").

3.	The driver’s area forward of the foremost padded barriers will permit the mounting of required safety equipment and vehicle operation equipment.

4.	Every school bus shall be constructed so that the noise level taken at the rear of the occupant nearest to the primary vehicle noise source shall not exceed eighty-five (85) dBA.

LAMPS AND SIGNALS

1.	Interior lamps shall be provided which adequately illuminate the aisle and the stepwell.  The stepwell light shall be illuminated by a service door operated switch so as to light only when the headlights and clearance lights are on and the service door is opened.

2.	Body instrument panel lights shall be controlled by a rheostat switch.

3.	School bus alternately flashing signal lamps:

a.	The bus shall be equipped with two (2) red lamps at the rear of the vehicle and two (2) red lamps at the front of the vehicle.

- In addition to the four (4) red lamps described above, four (4) amber lamps shall be installed so that one (1) amber lamp is located near each red signal lamp, at same level, but closer to vertical centerline of bus.  The system of red and amber signal lamps shall be wired so that the amber lamps are energized manually, and the red lamps are automatically energized (with the amber lamps being automatically de-energized) when the stop signal arm is extended or when the bus service door is opened. An amber pilot light and a red pilot light shall be installed adjacent to the driver controls for the flashing signal lamp to indicate to the driver which lamp system is activated.

- Area around the lens of each alternately flashing signal lamp and    extending outward approximately three inches (3") shall be black in

color. In installations where there is no flat vertical portion of body               immediately surrounding entire lens of  lamp, a circular or square band         of black approximately three inches (3") wide, immediately below and to        both sides of the lens, shall be black in color on body or roof area                 against which signal lamp is  seen (from distance of five hundred feet           (500')  along axis of the vehicle).  Visors or hoods, black in color, with           a minimum depth of four inches (4"), may be provided.

d.	Red lamps shall flash at any time the stop signal arm is extended.

e.	All flashers for alternately flashing red and amber signal lamps shall be enclosed in the body in a readily accessible location.

4.	Turn signal and stop/tail lamps:

a.	Bus body shall be equipped with amber rear turn signal lamps that are at least seven inches (7") in diameter or if a shape other than round, a minimum thirty-eight (38) square inches of illuminated area and meet specifications of the Society of Automotive Engineers.  These signal lamps must be connected to the chassis hazard warning switch to cause simultaneous flashing of the turn signal lamps when needed as a vehicular traffic hazard warning.  Turn signal lamps are to be placed as wide apart as practical and their centerline shall be approximately eight inches  (8") below the rear windows.  Type A conversion vehicle lamps must be at least twenty-one (21) square inches in the lens area and be in manufacturer's standard color.

b.	Buses shall be equipped with amber side-mounted turn signal lights.  The turn signal lamp on the left side shall be mounted rearward of the stop signal arm and the turn signal lamp on the right side shall be mounted rearward of the service door.

c.	Buses shall be equipped with four combination red stop/tail lamps:

- Two (2) combination lamps with a minimum diameter of seven inches

(7"), or if a shape other than round, a minimum of thirty-eight (38) square inches of illuminated area shall be mounted on the rear of the bus just inside the turn signal lamps.

- Two (2) combination lamps with a minimum diameter of four inches (4"), or if a shape other than round, a minimum of twelve (12) square inches of illuminated area shall be placed on the rear of the body between the beltline and the floor line.  The rear license plate lamps may be combines with one (1) lower tail lamp. Stop lamps shall be activated by the service brakes and shall emit a steady light when illuminated. Type A buses with bodies supplied by the chassis manufacturer may have the manufacturer's standard stop and tail lamps.

5.	Backup lamps

Bus body shall be equipped with two white rear backup lamp signals that are at least four inches (4") in diameter or, if a shape other than round, a minimum of thirteen (13) square inches of illuminated area, meeting SAE specifications.  If backup lamps are placed on the same line as the brake lamps and turn signal lamps, they shall be to the inside.

METAL TREATMENT

1.	All metal used in the construction of the bus body shall be zinc coated or aluminum coated or treated by an equivalent process before the bus is constructed.  Included are such items as structural members, inside and outside panels, door panels and floor sills.

Excluded are such items as door handles, grab handles, interior decorative parts and

other interior plated parts.

2.	All metal parts that will be painted shall be, in addition to the above requirements, chemically cleaned, etched, zinc-phosphate coated, and zinc-chromate or epoxy-primed or conditioned by an equivalent process.

3.	In providing for these requirements, particular attention shall be given to lapped surfaces, welded connections of structural members, cut edges, punched or drilled hole areas in sheet metal, closed or box sections, unvented or undrained areas, and surfaces subjected to abrasion during vehicle operation.

4.	As evidence that the above requirements have been met, samples of materials and sections used in construction of the bus body, when subjected to one thousand (1,000) hour salt spray test as provided for in latest revision of ASTM Standard B-117, "Standard Method of Salt Spray (Fog) Testing", shall not lose more than ten per cent (10%) of its material by weight.

MIRRORS

1.	The interior mirror shall be either clear view, laminated glass or clear view glass bonded to a backing which retains the glass in the event of breakage.  The mirror shall have rounded corners and protected edges.  Type A buses shall have a minimum of a six-inch by sixteen-inch (6" x 16") mirror and Type B, C and D buses shall have a minimum of a six-inch by thirty-inch (6" x 30") mirror.

2.	Each school bus shall be equipped with exterior mirrors meeting the requirements              of FMVSS - 111.  Mirrors shall be easily adjustable, but shall be rigidly braced so as            to reduce vibration.

3.	Mirrors shall be attached to the body in such a manner as to minimize fender 	breakage due to excessive vibration.

MOUNTING

1.	The chassis frame shall support the rear body cross member.  The bus body shall be attached to the chassis frame at each main floor sill, except where the chassis components interfere, in such a manner as to prevent shifting or separation of the body from the chassis under severe operating conditions.

2.	Insulation material shall be placed at all contact points between body and chassis frame and shall be so attached to the chassis frame or body that it will not move under severe operating conditions.

OVERALL LENGTH

The overall length of the bus shall not exceed forty-five feet (45’), excluding accessories.

OVERALL WIDTH

The overall width of the bus shall not exceed one hundred two inches (102"), excluding accessories.

REFLECTIVE MATERIAL

1.	Rear of bus body shall be marked with strips of reflective NSBY material to outline the perimeter of the back of the bus using material which conforms with the requirements of FMVSS 571.131 Table I.  The perimeter marking of rear emergency exits per FMVSS 217 and/or the use of reflective “SCHOOL BUS” signs partially accomplish the objective of this requirement.  To complete the perimeter marking of the back of the bus, strips of at least one and three-fourth inches (1 3/4") reflective NSBY material shall be applied horizontally above the rear windows and above the rear bumper extending from the rear emergency exit perimeter marking outward to the left and right rear corners of the bus; and vertical strips shall be applied at the corners connecting these horizontal strips.

2.	Sides of bus body shall be marked with reflective NSBY material at least one and three-fourth inches (1 3/4") in width, extending the length of the bus body and located (vertically) between the floor line and the beltline.

RUB RAILS

1.	There shall be one (1) rub rail located on each side of the bus approximately at the seat level which shall extend from the rear side of the entrance door completely around the bus body (except for the emergency door or any maintenance access door) to a point of curvature near the outside cowl on the left side.

2.	There shall be one (1) rub rail located approximately at floor line which shall cover the same longitudinal area as the upper rub rail, except at wheelhousing, and shall extend only to the radii of the right and left rear corners.

3.	Both rub rails shall be attached at each body post and all other up-right structural members.

4.	Both rub rails shall be four inches  (4") or more in width in finished form, shall be of 16-gauge steel or suitable material of equivalent strength, and shall be constructed in corrugated or ribbed fashion.

5.	Both rub rails shall be applied outside body or outside body posts.  Pressed-in or snap-on rub rails do not satisfy this requirement.  For Type A-1 vehicles using chassis manufacturer’s body, or for Types A-2, B, C and D buses using rear luggage or rear engine compartment, rub rails need not extend around rear corners.

6.	There shall be a rub rail or equivalent bracing located horizontally at the bottom edge of the body side skirts.

SEAT BELT FOR DRIVER

A Type two (2) lap belt/shoulder harness seat belt shall be provided for the driver.  The assembly shall be equipped with an emergency locking retractor (ELR) for the continuous belt system.  On all buses except Type A and Type C Cutaway equipped with standard chassis manufacturer’s driver’s seat, the lap portion of the belt shall be guided or anchored to prevent the driver from sliding sideways under it.  The lap belt/shoulder harness shall be designed to allow for easy adjustment in order to fit properly and effectively protect drivers varying from fifth (5th) percentile female to ninety-fifth (95th) percentile male.

SEAT AND CRASH BARRIERS

1.	All seats shall have a minimum depth of fifteen inches (15").  All seat backs shall be a minimum of twenty-four inches (24") high with a minimum twenty inches  (20") above seating reference point.

2.	In determining the seating capacity of the bus, the allowable average rump width           	shall be:

a.	Thirteen inches (13") where 3 - 3 seating plan is used.

b.	Fifteen inches (15") where 3 - 2 seating plan is used.

3.	All restraining barriers and passenger seats shall be constructed with materials that enable them to meet the criteria contained in the School Bus Seats Upholstery Fire Block Test.

4.	Each seat leg shall be secured to the floor by a minimum of two (2) bolts, washers and nuts.  Flange-head nuts may be used in lieu of nuts and washers, or seats may be track-mounted in conformance with FMVSS 222.  If track seating is installed, the manufacturer shall supply minimum and maximum seat spacing dimensions applicable to the bus, which comply with FMVSS 222.  This information shall be on a label permanently affixed to the bus.

5.	All seat frames attached to the seat rail shall be fastened with a minimum of two (2) bolts, washers and nuts or flange-headed nuts.

- All school bus bodies shall be equipped with restraining barriers conforming to FMVSS –222.

6.         Use of a flip seat at any side emergency door location in conformance with FMVSS-           222 and 217, including required aisle width to side door, is acceptable.  Any flip seat           shall be free of sharp projections on the underside of the seat bottom.  The                       underside of the flip-up seat bottoms shall be padded or contoured to reduce the               possibility of snagged clothing or injury during use.  Flip seats shall be constructed             to prevent passenger limbs from becoming entrapped between the seat back and              the seat cushion when in upright position.  The seat cushion shall be designed to               rise to a vertical position automatically when not occupied.

7.	School buses equipped with lap belts shall have the female end extend no more      than two inches from the seat bight.

STEPS

1.	The first step at the service door for all buses shall be not less than twelve inches (12") and not more than sixteen inches (16") from the ground, based on standard chassis specifications.

2.	Step risers shall not exceed a height of ten inches (10") .  When plywood is used on a steel floor or step, the riser height may be increased by the thickness of the plywood.

3.	The steps shall be enclosed to prevent accumulation of ice and snow.

4.	The steps shall not protrude beyond the side body line.

STEP TREADS

1.	All steps, including the floor line platform area, shall be covered with three-sixteenths inch (3/16") rubber floor covering or other materials equal in wear and abrasion resistance to top grade rubber.

2.	The step covering shall be permanently bonded to a durable backing material that is resistant to corrosion.

3.	A three-sixteenth inch (3/16") ribbed step tread shall have a one and one-half inch (1 1/2"), white nosing as an integral piece without any joint.

4.	The rubber portion of the step treads shall have the following characteristics:

a.	A special compounding for good abrasion resistance and high coefficient of friction.

b.	Flexibility so that it can be bent around a one-half inch (1/2") mandrel both at one hundred and thirty degrees Fahrenheit (130) and at twenty degrees Fahrenheit (20) without breaking, cracking or crazing.

c.	Show a durometer hardness of eighty-five (85) to ninety-five (95).

STIRRUP STEPS

There shall be at least one (1) folding stirrup step or recessed foothold and suitably located handles on each side of the front of the body for easy accessibility for cleaning the windshield and lamps except when windshield and lamps are easily accessible from the ground.  Steps are permitted in or on the front bumper, in lieu of the stirrup steps, if the windshield and lamps are easily accessible for cleaning from that position.

STOP SIGNAL ARM

There shall be an electric powered stop signal arm which complies with the requirements of FMVSS 131.

# STROBE LIGHT

Buses shall be equipped with a white, flashing strobe light to be installed on the roof of a school bus not to exceed one-third (1/3) the body length forward from the rear of the roof edge.  The light shall have a single clear lens emitting light three hundred and sixty degrees (360) around its vertical axis and may not extend above the roof more than the maximum legal height.  A manual switch and pilot light shall be included to indicate when the light is in operation.  Optionally the strobe light may be mounted on the roof in the area directly over the driver’s side crash barrier, and may be wired to activate with the amber alternately flashing signal lamps, continuing through the full loading or unloading cycle, with an override switch to allow activation of the strobe at any time for use in inclement weather.

SUN SHIELD

1.	Interior adjustable transparent sun shield not less than six inches by thirty-inches (6" x 30") for Types B, C and D vehicles, with a finished edge, shall be installed in a position convenient for use by the driver.

2.	On all Type A and Type C Cutaway buses the sun shield shall be manufacturer’s standard.

TAILPIPE

1.	Tailpipe shall extend out to but not more than two inches (2") beyond perimeter of the body in the case of the side exhaust and the bumper in the case of the rear exhaust.

2.	Tailpipe shall exit to the left or right of the emergency exit door in the rear of vehicle or to the left side of the bus.  Tailpipe shall not exit beneath any fuel filler location or beneath any emergency door.  All Types A and B may be manufacturer's standards.

TOW EYES OR HOOKS

Tow eyes or hooks shall be furnished on the rear and attached so they do not project beyond the rear bumper.  Tow eyes or hooks attached to the chassis frame shall be furnished by either the chassis or body manufacturer.  The installation shall be in accordance with the chassis manufacturer’s specifications.

UNDERCOATING

- The entire underside of the bus body, including the floor sections, cross member              and below floor line side panels, shall be coated with a rust-proofing compound for           which the compound manufacturer has issued a notarized certification for                        compliance to the bus body builder that the compound meets or exceeds all                     performance and qualitative requirements of paragraph 3.4 of Federal Specification           TT-C-520b, using modified test procedures* for the following requirements:

a.	Salt spray resistance - pass test modified to five percent (5%) salt and one thousand (1,000) hours

b.	Abrasion resistance - pass

c.	Fire resistance - pass

*Test panels to be prepared in accordance with paragraph 4.6.12 of TT-C-520b with   modified procedure requiring that test be made on a forty-eight (48) hour air cured   film at thickness recommended by the compound manufacturer.

2.	The undercoating compound shall be applied with suitable airless or conventional spray equipment to recommended film thickness and shall show no evidence of voids in cured film.

VENTILATION

1.	One auxiliary fan shall be required and meet the following requirements:

a.	Fan for left or right sides shall be placed in a location where they can be adjusted for maximum effectiveness and do not obstruct vision to any mirror.

b.	Fan shall be a nominal six inches  (6") diameter.

c.	Fan blades shall be covered with a protective cage.

d.	If additional auxiliary fans are installed, each fan shall be controlled by a separate switch.

2.	Body shall be equipped with a suitable controlled ventilating system of sufficient capacity to maintain proper quantity of air under operating conditions, without having to open windows except in extremely warm weather.

3.	Static-type non-closeable exhaust ventilation shall be installed in a low-pressure area of roof.

4.	Roof hatches designed to provide ventilation, regardless of the exterior weather conditions, may be provided.

WHEELHOUSING

1.	The wheelhousing opening shall allow for easy tire removal and service.

2.	The wheelhousing shall be attached to the floor sheets in such a manner to prevent any dust, water or fumes from entering the body.  The wheelhousing shall be constructed of a minimum 16-gauge steel.

3.	The inside height of the wheelhousing above the floor line shall not exceed twelve inches (12").

4.	The wheelhousing shall provide clearance for installation and use of tire chains on single and dual (if so equipped) power-driving wheels.

5.	No part of a raised wheelhousing shall extend into the emergency door opening.

WINDOWS

1.	Each full side window, other than emergency exits designated to comply with FMVSS 217, shall provide an unobstructed emergency opening of at least nine inches (9") but not more than thirteen inches (13") high and twenty-two inches  (22") wide, obtained by lowering window.  One side window on each side of the bus may be less than twenty-two inches (22") wide.

2.	Optional tinted and/or frost-free glazing may be installed in all doors, windows and windshields consistent with federal, state and local regulations.

# WINDSHIELD WASHERS

A windshield washer system shall be provided.

WINDSHIELD WIPERS

1.	A windshield wiping system, two (2)-speed or more, shall be provided.

2.	The wipers shall be operated by one (1) or more air or electric motors of sufficient power to operate wipers.  If one (1) motor is used, the wipers shall work in tandem to give full sweep of the windshield. Wiper blade shall not obstruct the driver’s view when in a parked position.

WIRING

1.	All wiring shall conform to current standards of the Society of Automotive Engineers.

2.	Circuits

- Wiring shall be arranged in circuits, as required, with each circuit protected by a fuse or circuit breaker.  A system of color and number coding shall be used and an appropriate identifying diagram shall be provided to the end user along with the wiring diagram provided by the chassis manufacturer.  The wiring diagrams shall be specific to the bus model supplied and include any changes to wiring made by the body manufacturer.  Chassis wiring diagrams shall also be supplied to the end user.  A system of color and number coding shall be used on buses.  The following body interconnecting circuits shall be color coded as noted:

| FUNCTION | COLOR |
| --- | --- |
| Left Rear Directional Light | Yellow |
| Right Rear Directional Light | Dark Green |
| Stoplights | Red |
| Back-up Lights | Blue |
| Taillights | Brown |
| Ground | White |
| Ignition Feed, Primary Feed | Black |

The color of cables shall correspond to SAE J 1128.

b.	Wiring shall be arranged in at least six (6) regular circuits, as follows:

1.	Head, tail, stop (brake) and instrument panel lamps

2.	Clearance and stepwell lamps (stepwell lamp shall be actuated when service door is opened)

3.	Dome lamp

4.	Ignition and emergency door signal

5.	Turn signal lamps

6.	Alternately flashing signal lamps

c.	Any of the above combination circuits may be subdivided into additional independent circuits.

d.	Whenever heaters and defrosters are used, at least one (1) additional circuit shall be installed.

e.	Whenever possible, all other electrical functions (such as sanders and  electric-type windshield wipers) shall be provided with independent and properly protected circuits.

f.	Each body circuit shall be coded by number or letter on a diagram of circuits and shall be attached to the body in a readily accessible location.

3.	The entire electrical system of the body shall be designed for the same voltage as the chassis on which the body is mounted.

4.	All wiring shall have an amperage capacity exceeding the designed load by at least twenty-five per cent (25%).  All wiring splices to be done at an accessible location and noted as splices on wiring diagram.

5.	A body wiring diagram of easily readable size shall be furnished with each bus body or affixed in an area convenient to the electrical accessory control panel.

6.	Body power wire shall be attached to a special terminal on the chassis.

7.	All wires passing through metal openings shall be protected by a grommet.

8.	Wires not enclosed within the body shall be fastened securely at intervals of not more than eighteen inches  (18").  All joints shall be soldered or jointed by equally effective connectors, which shall be water-resistant and corrosion-resistant.

SPECIALLY EQUIPPED SCHOOL BUS SPECIFICATIONS INTRODUCTION

Equipping buses to accommodate students with disabilities is dependent upon the needs of the passengers.  While one bus may be fitted with a lift, another may have belts installed to secure child seats.  Buses so equipped are not to be considered a separate class of school bus, but simply a regular school bus that is equipped for special accommodations.

The specifications in this section are intended to supplement specifications in the chassis and body sections.  In general, specially equipped buses shall meet all the requirements of the preceding sections, plus those listed in this section.  It is recognized that the field of special transportation is characterized by varied needs for individual cases and by rapidly emerging technologies for meeting individual student needs.  A flexible, “common-sense” approach to the adoption and enforcement of specifications for these vehicles, therefore, is prudent.

As defined by 49 Code of Federal Regulations (CFR) §571.3, “Bus means a motor vehicle with motive power, except a trailer, designed for carrying more than ten persons” ( eleven or more including the driver).  This definition also embraces the more specific category, school bus .   Vehicles with ten or fewer occupant positions (including the driver) are not classified as buses.  For this reason, the federal vehicle classification multipurpose passenger vehicle (49 CFR § 571.3), or MPV, must be used by manufacturers for these vehicles in lieu of the classification school bus.  The definition of designated seating position in 49 CFR § 571.3 states that, in the case of “vehicles sold or introduced into interstate commerce for purposes that include carrying students to and from school or related events” and which are “intended for securement of an occupied wheelchair during vehicle operations,” each wheelchair securement position shall be counted as four designated seating positions when determining the classification (whether school bus or MPV).  This classification system does not preclude state or local agencies or these national specifications from requiring compliance of school bus-type MPVs with the more stringent federal standards for school buses.  The following specifications address modifications as they pertain to school buses that, with standard seating arrangements prior to modification, would accommodate  eleven or more occupants including the driver. If by addition of a power lift, wheelchair positions or other modifications, the capacity is reduced such that vehicles become MPVs, the intent of these specifications is to require these vehicles to meet the same specifications they would have had to meet prior to such modifications, and such MPVs are included in all references to school buses and requirements for school buses which follow.

DEFINITION

A specially equipped school bus is any school bus that is designed, equipped and/or modified to accommodate students with special transportation needs.

GENERAL REQUIREMENTS

- Specially equipped school buses shall comply with the National School Transportation Specification & Procedures and with the Federal Motor Vehicle Safety Standards (FMVSS) applicable to their Gross Vehicle Weight Rating (GVWR) category.

- Any school bus to be used for the transportation of children who utilize a wheel- chair or other mobile positioning device, or who require life-support equipment that prohibits use of the regular service entrance, shall be equipped with a power lift, unless a ramp is needed for unusual circumstances related to passenger needs.

AISLES

All school buses equipped with a power lift shall provide a minimum 30-inches aisle leading from any wheelchair position to at least one emergency exit door. A wheelchair securement position shall never be located directly in front of (blocking) a power lift door location.

# GLAZING

Tinted glazing may be installed in all doors, windows and windshields consistent with federal, state and local regulations.

IDENTIFICATION

Specially equipped school buses shall display the International Symbol of Accessibility below the window line.  Such emblems shall be white on blue or black background, shall not exceed 12 inches square in size and shall be on a high-intensity retroreflective material meeting the requirements of Federal Highway Administration (FHWA) FP-85, Standard Specifications for Construction of Roads and Bridges on Federal Highway Projects.

PASSENGER CAPACITY RATING

In determining the passenger capacity of a school bus for purposes other than actual passenger load (e.g., vehicle classification or various billing/reimbursement models), any location in a school bus intended for securement of a wheelchair during vehicle operation shall be regarded as four designated seating positions, and each lift area shall count as  four designated seating positions.

POWER LIFTS AND RAMPS

A.	The power lift shall be located on the right side of the bus body.  Exception: The lift may be located on the left side of the bus if, and only if, the bus is only used to deliver students to the left side of one-way streets.

1.	A ramp device may be used in lieu of a mechanical lift if the ramp meets all the requirements of the Americans with Disabilities Act (ADA) as found in 36 CFR § 1192.23, Vehicle ramp.

2.	A ramp device that does not meet the specifications of ADA, but does meet the specifications of paragraph C of this section, may be installed and used, when, and only when, a power lift system is not adequate to load and unload students having special and unique needs.  A readily accessible ramp may be installed for emergency exit use.  If stowed in the passenger compartment, the ramp must be properly secured and  placed away from general passenger contact.  It must not obstruct or restrict any aisle or exit while in its stowed or deployed position.

3.	All specially equipped school buses shall provide a level-change                mechanism or boarding device (e.g., lift or ramp), complying with paragraph B or C of this section, with sufficient clearances to permit a wheelchair user to reach a securement location.

B.	Vehicle lift and installation

- General: Vehicle lifts and installations shall comply with the requirements set      forth in FMVSS 403, Platform Lift Systems for Motor Vehicles, and FMVSS           404, Platform Lift Installations in Motor Vehicles.

- Design loads.  The design load of the lift shall be at least 800 pounds.  Working parts, such as cables, pulleys and shafts, which can be expected to wear, and upon which the lift depends for support of the load, shall have a safety factor of at least six, based on the ultimate strength of the material.  Non-working parts, such as platform, frame and attachment hardware would not be expected to wear, shall have a safety factor of at least three, based on the ultimate strength of the material.

- Lift capacity: The lifting mechanism and platform shall be capable of                  operating effectively with a wheelchair and occupant mass of at least 800          pounds.

- Controls: (See 49 CFR 571. 403, S6.7, Control systems.)

- Emergency operations: (See 49 CFR 571.403, S6.9, Backup operation.)

- Power or equipment failures: (See 49 CFR 571.403, S6.2.2, Maximum               platform velocity.)

- Platform barriers: (See 49 CFR 571. 403, S6.4.7, Wheelchair retention.)

- Platform surface: (See 49 CFR 571.403, S6.4.2, S6.4.3, Platform                       requirements.) (See, also “Wheelchair or Mobility Aid Envelope” figure at the      end of this subsection.)

- Platform gaps and entrance ramps: (See 49 CFR 571.403, S6.4.4, Gaps,            transitions and openings.)

- Platform deflection: (See 49 CFR 571. 403, S6.4.5, Platform deflection.)

- Platform movement: (See 49 CFR 571. 403, S6.2.3, Maximum platform              acceleration.)

- Boarding direction:	The lift shall permit both inboard and outboard facing of      wheelchair and mobility aid users.

- Use by standees: Lift shall accommodate persons who are using walkers,           crutches, canes or braces, or who otherwise have difficulty using steps. The   	perform may be marked to indicate a preferred standing position. Note: This      item refers to equipment specifications. (Also see section, TRANSPORTATION      FOR STUDENTS WITH DISABILITIES AND SPECIAL HEALTH CARE NEEDS,         Subsection D, Special Equipment Use and Operation, for applicable                    operational procedures stating that “During lift operations (including manual)      no one shall be allowed to stand on the lift platform.”)

- Handrails: (See 49 CFR 571.403, S6.4.9, Handrails.)

- Circuit breaker: A resettable circuit breaker shall be installed between the           power source and the lift motor if electrical power is used. It shall be located      as close to the power source as possible, but not within the passenger/driver      compartment.

- Excessive pressure: (See 49 CFR 571. 403, S6.8, Jacking prevention.)

- Documentation: The following information shall be provided with each               vehicle equipped with a lift.

(1)		A phone number where information can be obtained about installation, repair and parts. (Detailed written instructions and a parts list shall be available upon request.)

(2)		Detailed instructions regarding use of the lift shall be readily visible when the lift door is open, including a diagram showing the proper placement and positioning of wheelchair/mobility aids on the lift.

- Training materials:	The lift manufacturer shall make training materials              available to ensure the proper use and maintenance of the lift. These may          include instructional videos, classroom curriculum, system test results or            other related materials

- Identification and certification: Each lift shall be permanently and legibly            marked or shall incorporate a non-removable label or tag that states it               conforms to all applicable requirements of the current National School                Transportation Specifications and Procedures. In addition and upon request        of the original titled purchaser, the lift manufacturer or an authorized                 representative shall provide a notarized Certificate of Conformance, either          original or photocopied, which states that the lift system meets all the                applicable requirements of the current National School Transportation                Specifications and Procedures.

C.	Vehicle ramp

- If a ramp is used, it shall be of sufficient strength and rigidity to support the special device, occupant and attendant(s). It shall be equipped with a protective flange on each longitudinal side to keep the special device on the ramp.

- The surface of the ramp shall be constructed of non-skid material.

- The ramp shall be equipped with handles and shall be of weight and design to permit one person to put the ramp in place and return it to its storage place.

- Ramps used for emergency evacuation purposes may be installed in raised floor buses by manufactures. They shall not be installed as a substitute for a lift when a lift is capable of serving the need.

REGULAR SERVICE ENTRANCE

- On power lift-equipped vehicles, steps shall be the full width of the step well, excluding the thickness of the doors in the open position.

- A suitable device shall be provided to assist passengers during ingress and egress. This device shall allow for easy grasping or holding and shall have no openings or pinch points that might entangle clothing, accessories or limbs.

RESTRAINING DEVICES

- On power lift-equipped school buses with a GVWR of 10,000 pounds or more, seat frames may be equipped with attachment points to which belt assemblies can be attached for use with child safety restraint systems (CSRSs) that comply with FMVSS No. 213, Child Restraint Systems. Any belt assembly anchorage shall comply with FMVSS No. 210, Seat Belt Assembly Anchorages.

- Alternatively, a child restraint anchorage system that complies with FMVSS No. 225, Child Restraint Anchorage Systems, may be installed.

- Seat belt assemblies, if installed, shall conform to FMVSS No. 209, Seat Belt Assemblies.

- Child safety restraint systems, which are used to facilitate the transportation of children who in other modes of transportation would be required to used a child, infant or booster seat, shall conform to FMVSS No. 213.

# SEATING ARRANGEMENTS

Flexibility in seat spacing to accommodate special devices shall be permitted to meet passenger requirements.  All seating shall meet the requirements of FMVSS No. 222, School Bus Passenger Seating and Crash Protection.

SECUREMENT AND RESTRAINT SYSTEM FOR WHEELCHAIRS AND WHEELCHAIR- SEATED OCCUPANTS

For purposes of understanding the various aspects and components of this section, the term securement and tiedown and the phrases securement system or tiedown system are used exclusively in reference to the devices that anchor the wheelchair to the vehicle. The term restraint and the phrase restraint system  are used exclusively in reference to the equipment that is intended to limit the movement of the wheelchair occupant in a crash or sudden maneuver. The term wheelchair tiedown and occupant restraint system (WTORS)  is used to refer to the total system that secures the wheelchair and restrains the wheelchair occupant.

A.	 WTORS-general requirements:

- The wheelchair tiedown and occupant restraint system installed in specially equipped school buses shall be designed, installed, and operated for use with forward-facing wheelchair-seated passengers and shall comply with all applicable requirements of FMVSS 222, School Bus Passenger Seating and Crash Protection, and SAE J2249, Wheelchair Tiedown and Occupant Restraint Systems for Use in Motor Vehicles.¹

- The WTORS, including the anchorage track, floor plates, pockets or other anchorages, shall be provided by the same manufacturer or shall be certified to be compatible by manufactures of all equipment/systems used.

- Wheelchair securement positions shall be located such that wheelchairs and their occupants do not block access to the lift door.

- A device for storage of the WTORS shall be provided. When the system is not in use, the storage device shall allow for clean storage of the system, shall keep the system securely contained within the passenger compartment, shall provide reasonable protection from vandalism and shall enable the system to be readily accessed for use.

- The WTORS, including the storage device, shall meet the flammability standards established in FMVSS No. 302, Flammability of Interior Materials.

- The following information shall be provided with each vehicle equipped with a securement and restraint system:

  - A phone number where information can be obtained about installation, repair and parts. (Detailed written instructions and a parts list shall be available upon request.)

  - Detailed instructions regarding use, including a diagram showing the proper placement of the wheelchair/mobility aids and positioning of securement devices and occupant restraints, including correct belt angles.

- The WTORS manufacturer shall make training materials available to ensure the        proper use and maintenance of the WTORS. These may include instructional videos, classroom curriculum, system test results or other related materials.

B.	Wheelchair Securement/Tiedown: (See 49 CFR 571.403, S5.4.1, S5.4.2.)

Each wheelchair position in a specially equipped school bus shall have a minimum clear floor area of 30 inches laterally by 48 inches longitudinally. Additional floor area may be required for some wheelchairs. Consultation between the user and the manufacturer is recommended to ensure that adequate area is provided.

C.	Occupant restraint system: (See 49 CFR 571.403, S5.4.3, S5.4.4.)

SPECIAL LIGHT

Doorways in which lifts are installed shall be equipped with a special light that provides a minimum of two foot-candles of illumination measured on the floor of the bus immediately adjacent to the lift during lift operation.

# SPECIAL SERVICE ENTRANCE

- Power lift-equipped bodies shall have a special service entrance to                                    accommodate the power lift. Exception: A special service entrance shall not be                required if the lift is designed to operate within the regular service entrance, is                  capable of stowing such that the regular service entrance is not blocked in any way            and a person entering or exiting the bus is not impeded in any way.

B.  The special service entrance and door shall be located on the right                     	         side of the bus and shall be designed so as not to obstruct the                                         regular service entrance. Exception: A special service entrance and                                 door may be located on the left side of the bus only if the bus is used                               only to deliver students to the left side of one-way streets and its                                     use is limited to that function.

C. The opening may extend below the floor through the bottom of the body  skirt.                  If such an opening is used, reinforcements  shall be installed at the front and                    rear of the floor opening to support the floor and give the same strength as                       other floor openings.

D. A drip molding shall be installed above special service entrance to effectively                     divert water from the entrance.

E. Door posts and headers at the special service entrance shall be reinforced                           sufficiently to provide support and strength equivalent to the areas of the side                   of the bus not used for special service entrance.

SPECIAL SERVICE ENTRANCE DOORS

A.    A single door or double doors may be used for the special service entrance.

B.    A single door shall be hinged to the forward side of the entrance unless this would  obstruct the regular service entrance.  If the door is hinged to the rearward side of  the doorway, the door shall utilize a safety mechanism that will prevent the door     from swinging open should the primary door latch fail.  If double doors are used,    the system shall be designed to prevent the door(s) from being blown open by the  aerodynamic forces created by the forward motion of the bus, and/or  shall            incorporate a safety mechanism to provide secondary protection should the            primary latching mechanism(s) fail.

C.    All doors shall have positive fastening devices to hold doors in the “open”  position   when the special service entrance is in use.

D.    All doors shall be weather sealed.

E.	When manually operated dual doors are provided, the rear door shall have at least a  one-point fastening device to the header.   The forward-mounted door shall have at least three one-point fastening devices.  One shall be to the header, one to the floor line of the body, and the other shall be into the rear door.  The door and hinge mechanism shall have strength that is greater than, or equivalent to, the strength of the emergency exit door.

F.     Door materials, panels and structural components shall have strength equivalent to

the conventional service and emergency doors.  Color, rub rail extensions, lettering and other exterior features shall match adjacent sections of the body.

G.    Each door shall have windows set in a waterproof manner that are visually similar in size and location to adjacent non-door windows.  Glazing shall be of the same type and tinting (if applicable) as standard fixed glass in other body locations.

H.  Door(s) shall be equipped with a device that will actuate an audible or               flashing signal located in the driver’s compartment when door(s) is not                    securely closed and the ignition is in the “on” position.

I.    A switch shall be installed so that the lift mechanism will not operate when the lift  platform door(s) is closed.

J.      Special service entrance doors shall be equipped with padding at the top edge of the door opening.  The padding shall be at least three inches wide and one inch thick and shall extend the full width of the door opening.

# SUPPORT  EQUIPMENT AND ACCESSORIES

A.	Each specially equipped school bus that is set up to accommodate wheelchairs or other assistive or restraint devices with belts attached shall contain at least one webbing cutter properly secured in a location within reach of the driver while belted into his/her driver’s seat.  The belt cutter shall be durable and designed to prevent the operator or others from being cut during use.

B.	Special equipment or supplies that are used in the bus for mobility assistance, health support or safety purposes shall meet any local, federal and engineering standards that may apply, including requirements for proper identification.

Equipment that may be used for these purposes includes, but is not limited to:

1. Wheelchairs and other mobile seating devices.  (See subsection on         Securement and Restraint System for Wheelchairs and Wheelchair-seated Occupants.)

- Crutches, walkers, canes and other ambulating devices to assist                 ambulation.

- Medical support equipment.  This may include respiratory devices, such

as oxygen bottles (which should be no larger than 22 cubic feet for                  liquid oxygen and 38 cubic feet for compressed gas), or ventilators.                  Tanks and valves should be located and positioned to protect them                  from direct sunlight, bus heater vents or other heat sources. Other                   equipment may include intravenous and fluid drainage apparatus.

C.	All portable equipment and special accessory items, including the equipment listed above, shall be secured at the mounting location to withstand a pulling force of five times the weight of the item or shall be retained in an enclosed, latched compartment.  The compartment shall be capable of withstanding forces applied to its interior equal to five times the weight of its contents without failure of the box’s integrity and securement to the bus.  Exception: If these specifications provide specific requirements for securement of a particular type of equipment (e.g., wheelchairs), the specific specification shall prevail.

TECHNOLOGY AND EQUIPMENT, NEW

It is the intent of these specifications to accommodate new technologies and equipment that will better facilitate the transportation of students with special needs.  New technology and equipment is acceptable for use in specially equipped vehicles if:

A.	It does not compromise the effectiveness or integrity of any major safety system. (Examples of safety systems include, but are not limited to, compartmentalization, the eight-light warning system, emergency exits and the approved color scheme.)

B.	It does not diminish the safety of the bus interior.

C.	It does not create additional risk to students who are boarding or exiting the bus or are in or near the school bus loading zone.

D.	It does not require undue additional activity and/or responsibility for the driver.

E.	It generally increase efficiency and/or safety of the bus, generally provides for a safer or more pleasant experience for the occupants and pedestrians in the vicinity of the bus, and/or generally assists the driver and makes his/her many tasks easier to perform.

MINIMUM STANDARDS FOR OPTIONAL EQUIPMENT

ADDITIONAL HEATER(S)

School buses may be equipped with more than one (1) heater.  If more than one (1) heater is used, additional heaters may be of re-circulating air type.  All heaters must meet the standards specified for heaters in the minimum standards for school bus bodies. Heater booster pump shall be required with rear heater.

AIR BRAKES

Air brakes are optional on Type B, Type C, and Type D buses of less than 71 capacity. Air brakes require an air dryer and automatic moisture ejector.

AIR CONDITIONER

1.	SPECIAL REQUIREMENTS - The following specifications apply to all school buses equipped with air conditioning:

a.	ALTERNATOR - Type A buses equipped with air conditioning shall be furnished with an alternator with a minimum output rating of one hundred thirty (130) amperes.  Types B, C and D buses equipped with air conditioning shall be furnished with an alternator with a minimum output rating of one hundred seventy-five (175) amperes.  Type A buses equipped with air conditioning and wheelchair lifts shall be furnished with manufacturer's largest optional output alternator.  Types B, C and D buses equipped with air conditioning and wheelchair lifts shall be furnished with alternators with a minimum output rating of one hundred seventy-five (175) amperes.

b.	INSULATION - Minimum five (5) ply nominal five-eighth inch (5/8") thickness plywood or material of equal strength and insulation R value that meets or exceeds properties of exterior-type softwood plywood, C-D grade as specified in standard issued by the U. S. Department of Commerce shall be installed over the existing or manufacturer's standard steel floor for insulation.  Air conditioned buses shall have the equivalent of one point five inches  (1.5") of fiberglass or other insulation in the ceilings and walls, including the interior of hat-shaped bows.  The insulation shall have a minimum R-factor value of 5.77.

c.	TINTING - All glass (windshield, service and emergency doors, side and rear windows) shall be equipped with maximum tinting allowed by Federal and A.N.S.I. standards for the respective locations, except that windows rear of the driver's compartment shall have approximately twenty-eight percent (28%) light transmission.

d.	WHITE ROOF - The roof of the bus shall be painted white extending down               to within 6 inches (6") above the drip rails on the sides of the body, except             that front and rear roof caps shall remain yellow.

e.	EXTRA COOLING - When so specified, additional cooling may be ordered for sixteen (16) through eighty-nine (89) passenger school buses.  This is intended for use in buses operated under severe conditions (e.g., buses with handicapped lifts where the doors  remain open for long periods of time, buses operated in urban areas with slow, stop-and-go traffic, etc.).  Ordering this option will provide a BTU/hr capacity equal to the next passenger-capacity category, as shown in Table 1.  (For example, an eighteen [18] passenger school bus with this option would be furnished with a fifty-three thousand [53,000] BTU/hr capacity air conditioning system instead of the standard forty thousand [40,000] BTU/hr unit).

2.	GENERAL AND PERFORMANCE REQUIREMENTS

Air-conditioning systems furnished to meet the requirements of this specification shall be the mechanical vapor compression refrigeration type.  Each air conditioning system shall have sufficient power for simultaneous cooling, circulating, cleaning and dehumidifying the air.  The refrigerant for the system must be non-toxic, non-flammable and non-explosive.  The air conditioning system shall be manufactured to conform to acceptable practices and procedures to accomplish design specifications.  Air conditioning units furnished under this specification shall be of the current year's production.  Details not specifically defined herein shall be in accordance with the manufacturer's standard commercial practice for products of this type.  Table 1 lists the components and the appropriate rating required by this specification:

TABLE 1

| BUS SIZE | CAPACITY BTU/HR | AIR FLOW CFM | COMPRESSOR(S) NUMBER | CONDENSER(S) LOCATION/NO. | EVAPORATOR(S) LOCATION/NO. |
| --- | --- | --- | --- | --- | --- |
| 16 – 24 PASS | 40,000 | 1,200 | 1 | 1 - skirt mtd. | 1 - rear |
| 25 – 47 PASS | 53,000 | 1,900 | 1 | 1 | 1 - rear |
| 48 – 71 PASS* | 84,000 | 2,000 | 2 | 2 | 2 - 1 each side |
| 72 – 89 PASS* | 108,000 | 2,400 | 2 | 2 | 2 - 1 each side |

* Except rear engine buses may be single units provided they meet or exceed         the BTU/CFM requirements.

3.	GENERAL SPECIFICATIONS

a.	CONTROLS

A control box or panel, which shall be located in the driver's compartment, shall be permanently installed to house inside temperature and fan speed(s) controls.  The control box or panel shall be positioned so that the driver shall be able to operate the air conditioning controls while seated in the driver's seat and operating the bus.  The fan(s) (blower) shall have a minimum of two operating speeds ("off" is not considered an operating speed).

b.	INSTALLATION

1.	Installing Dealer - Installation of the air conditioning system(s) shall be by the bus body company or by an authorized factory air conditioning dealer who normally stocks, sells, installs and services a unit of the type being furnished.

2.	Workmanship - Poor, shoddy installation will be grounds for immediate rejection of the complete bus.

NOTE:  NO INSTALLATION OF ANY AIR CONDITIONING UNITS OR SYSTEMS SHALL, UNDER ANY CIRCUMSTANCES, VOID THE CHASSIS MANUFACTURER'S ENGINE WARRANTY.

4.	COMPONENTS

The following is a list of components required for air conditioning systems.  Any parts or components not specifically mentioned below, but which are required to provide a complete operating unit, or which are standard for the model offered, shall be included:

a.	BLOWER UNIT - The blower unit shall be of heavy-duty, commercial design and shall circulate air over the evaporator(s) to cool the passenger compartment.  Fans shall be of the centrifugal or axial type and quiet in operation.  Unless they are self-contained, fan motor(s) shall have bearings of the permanent lubrication type and designed to operate on the 12V-DC system of the school bus.

b.	COMPRESSORS - Compressors shall be of the air conditioning or chassis manufacturer's standard design.  Lubrication of all moving parts shall be accomplished automatically.  An automatic (electric) clutch shall be provided on each compressor.  The compressor size shall be as required to meet the

performance requirements above.  Compressor(s) shall be compatible with the engine speed.

NOTE:  Compressors shall be geared so that their speed does not exceed the manufacturer's maximum recommended sustained speeds at a road speed of sixty (60) mph in high gear.

c.	CONDENSERS - The condenser(s) shall be recommended by the manufacturer of the unit.  The air conditioning manufacturers shall use their standard condenser fabrication and installation practices.

d.	EVAPORATOR (COOLING COIL) - Air conditioning manufacturers shall use their standard cooling coil, fabrication and installation practices.

e.	REFRIGERATOR DRYER - A dryer with a minimum of ten (10) ounces of desiccant shall be installed in the refrigerating circuit.  The system shall be designed and installed in accordance with the manufacturer's standard practice to insure optimum performance and ease of service/replacement.

5.	CERTIFICATION

In the event the bus air conditioning system fails to meet or exceed all conditions and requirements of this specification, the cost of any modifications to comply with design specifications shall be borne by the supplier.

AUXILIARY FAN

Additional auxiliary fans may be installed and shall comply with the specifications listed in the body section (Ventilation)

DOOR LOCK

A system may be installed for locking the service door and emergency door when the bus is not in use.  It shall be designed so that the bus will not start when the emergency door is locked.

DRIVER'S SEAT - HIGHBACK TYPE

The driver's seat may be of the highback type with a minimum seat back adjustment of fifteen degrees (15) and with a head restraint to accommodate a ninety-five percentile (95%) adult male (95 percentile adult male as defined in FMVSS-208).  The driver's seat shall be secured with nuts, bolts and washers or flanged-headed nuts.

EXTENDED WARRANTY

The bus may be purchased with a chassis extended warranty.  The warranty shall be the manufacturer's standard.

FLAT FLOOR

1.	The floor shall be flat with no wheelhouse intrusions.  The flat floor area shall extend from thirty (30) inches rearward of the driver's seating reference point to the rear wall of the bus and across the width of the bus to within five (5) inches of the left and right walls.

2.	Within the area specified in paragraph one (1) of this section, the floor's structural             rigidity shall be approximately equal throughout.

FUEL TANK - SIXTY GALLON

A sixty-gallon (60 gal.)  fuel tank, with an eighty-three percent (83%) actual draw capacity, may be used on Type B, C, and D Mid-range (fifty-three (53) capacity or less) school buses. The tank shall meet all requirements specified under the Fuel Tank in the chassis section.

# HEAT BOOSTER PUMP

A heat booster pump may be used on all Type A, B, C and D school buses.

POWER LIFT AND RAMP

School buses may be equipped with a power lift or ramp.  For specifications, see Power Lifts and Ramps, Specially Equipped School Buses Section.

PUBLIC ADDRESS SYSTEMS

Buses may be equipped with a public address system having interior and exterior speakers. No internal speakers other than the driver’s communication system maybe installed within four (4) feet of the driver’s back seat in it’s rear most upright position.

REFLECTIVE MATERIAL

Front and/or rear bumper may be marked diagonally forty-five degrees (45º) down to centerline of pavement with two inches (2”)+or – one-fourth inch (1/4”) wide strips of contrasting reflective material.

RESTRAINING DEVICES

1.	On power-lift equipped vehicles, seat frames may be equipped with attachments or devices to which belts, restraining harnesses or other devices may be attached.  Attachment framework or anchorage devices, if installed, shall conform to FMVSS 210.

2.	Seat belt assemblies, if installed, shall conform to FMVSS 209.

3.	Child restraint systems, which are used to facilitate the transportation of                  children who in other modes of transportation would be required to use a                child, infant or booster seat, shall conform to FMVSS 213 and 222. Maximum           seat spacing would be required with a rear facing car seat.

STOP ARM, AIR OPERATED

An air operated stop arm is optional on Type B, C and D buses.  The stop arm shall comply with FMVSS.

STORAGE BOX

If tools, tire chains and/or tow chains are carried on the bus, a container of adequate strength and capacity may be provided.  Such storage container may be located either inside or outside the passenger compartment but, if inside, it shall have a cover (seat cushion may not serve as this purpose) capable of being securely latched and be fastened to the floor convenient to either the service or emergency door.

STORAGE COMPARTMENT, LUGGAGE

If luggage and/or equipment is to be carried on the bus, a compartment of adequate strength and capacity may be provided.  The compartment shall be side mount underbody type with flush mount door and recessed handle, and may range from ten (10) to thirty (30) cubic feet in size.

TINTED GLAZING

Tinted and/or frost-free glazing may be installed in all doors, windows and windshields consistent with Federal, State and local regulations.

WHITE ROOF

The roof of the bus maybe painted white extending down to within six inches (6") above the drip rails on the sides of the body, except that front and rear roof caps shall remain yellow.

MINIMUM STANDARDS

PUPIL TRANSPORTATION SERVICE VEHICLES

Pupil transportation service vehicles shall be standard production models and shall be equipped with adequate springs, axles, tires, and other load carrying components to meet GVWR requirements.  Optional equipment may be purchased for service vehicles provided they appear as part of the specifications used by the respective school districts in soliciting bids on service vehicles.

ALTERNATIVE FUELS

INTRODUCTION

This standard is designed to be used as an overview of the alternative fuels being utilized for school transportation.  The standard is not designed to replace current applicable federal, state, manufacturing or safety standards that may exceed requirements within this standard.  There will be advancements in engineering and improvements in equipment fabrication methods and operating practices that differ from those specifically called for in this standard.  Such deviations or improvements may provide safety and may meet the intent of and be compatible with this standard.  Entities wishing to purchase alternative fuel school buses should use this section only as a starting point.  More detailed specifications, including specific design and performance criteria and safety standards, should be researched by prospective purchasers of alternative fuel school buses.

GENERAL REQUIREMENTS

Alternative fuel school buses shall meet the following requirements:

1.	Chassis shall meet all standards previously mentioned in BUS CHASSIS STANDARDS.

2.	Chassis shall meet all applicable FMVSS standards.

3.	Fuel system integrity shall allow zero (0) leakage when impacted by a mobile barrier in accordance with test conditions specified in FMVSS 301 or FMVSS 303, as applicable.

4.	Original equipment manufacturers (OEMs) and conversion systems using compressed natural gas (CNG) shall comply with NFPA Standard 52 “Compressed Natural Gas Vehicular Fuel Systems” in effect at the time of installation.  Fuel systems using liquefied petroleum gas (LPG) shall comply with the NFPA Standard 58 “Liquefied Petroleum Gases Engine Fuel Systems” in effect at the time of installation.

5.	All alternative fuel buses shall travel a loaded range of not less than 200 miles, except those powered by electricity which shall travel not less than 80 miles.

6.	Natural gas-powered buses shall be equipped with an interior/exterior gas detection system.  All natural gas-powered buses shall be equipped with a fire detection and suppression system.

7.	All materials and assemblies used to transfer or store alternative fuels shall be installed outside the passenger/driver compartment.

8.	All Type C and D buses using alternative fuel shall meet the same base requirements of BUS CHASSIS STANDARDS for Power and Grade ability, i.e., at least one published net horsepower per each 185 pounds of GVWR.

9.	The total weight shall not exceed the GVWR when loaded to rated capacity.

10.	The manufacturer supplying the alternative fuel equipment must provide the owner and operator with adequate training and certification in fueling procedures, scheduled maintenance, troubleshooting, and repair of alternative fuel equipment.

11.	All fueling equipment shall be designed specifically for fueling motor vehicles and shall be certified by the manufacturer as meeting all applicable federal, state and industry standards.

12.	All on-board fuel supply containers shall meet all appropriate requirements of the ASME code, the DOT regulations, or applicable FMVSS and NFPA Standards.

13.	All fuel supply containers shall be securely mounted to withstand a static force of 8 times their weight in any direction.

14.	All safety devices that may discharge to the atmosphere shall be vented to the outside of the vehicle.  The discharge line from the safety relief valve on all school buses shall be located in a manner appropriate to the characteristics of the alternative fuel.  Discharge lines shall not pass through the passenger compartment.

- A positive quick acting (1/4 turn) shut-off control valve shall be installed in the gaseous fuel supply lines as close to the fuel supply containers as possible.  The controls for this valve shall be placed in a location easily operable from the exterior of the vehicle.  The location of the valve control shall be clearly marked on the exterior surface of the bus.

16.	A grounding system shall be required for grounding of the fuel system during maintenance related venting.

CHARACTERISTICS OF ALTERNATIVE FUELS

For the purpose of this standard, alternative fuels refer to the specific fuels listed below. A brief description of each fuel and the advantages and disadvantages of each fuel are shown.

Note:	There are two other more exotic fuels being examined, hydrogen and solar power. These two energy sources are in their infancy as alternative fuels for motor vehicles, and are not covered within the scope of this document.

1.	Liquid alternative fuels

a.	Methanol

Methanol, a liquid at normal ambient temperatures, is colorless, and is made primarily from natural gas or coal.  Extensive experiments have been conducted with auto and truck engines powered by methanol.  There are a number of urban transit bus fleets currently using methanol; California has experience with methanol as an alternative fuel for school buses through their School Bus Demonstration Project.

(1)	Advantages:

(a)	The principal advantage to methanol is that the emissions produced are quite low in particulates and NOx.

(b)	Another major advantage is that it mixes with gasoline and can be used as M85 which is fifteen percent (15%) gasoline and eighty-five percent (85%) Methanol.  Also, flexible fuel vehicles run on a blend of the two fuels.

(c)	Methanol has a high cetane rating which assists diesel engine performance.

(d)	Methanol is biodegradable and readily assimilates with water.

(e)	Methanol burns smokeless.

(f)	Methanol is a domestically produced energy source.

(2)	Disadvantages:

(a)	Methanol is corrosive, particularly to aluminum; engines and fuel systems specially designed to handle it use different materials, such as stainless steel.

(b)	Methanol has less than half the power per equivalent gallon (BTU value) as that of diesel fuel.  For an equivalent range, this requires storage tanks twice the size of diesel tanks.

(c)	Methanol is quite toxic.  Direct exposure to the human body has the potential of causing blindness and kidney failure.  Since it is tasteless and colorless, it cannot be easily detected should it get into a water supply.

(d)	Methanol combustion generates high amounts of formaldehyde, a potential cancer causing substance.  This can be offset with exhaust after-treatment, such as special catalytic converters.

(e)	In its pure state, methanol burns with a colorless flame, so a fire is hard to see.  In addition, it is highly volatile and has a relatively low flash point.

- The distribution system and infrastructure for methanol fueling are considerably less widespread than for gasoline and diesel.

b.	Ethanol

Ethanol is a distilled agricultural alcohol product that is a liquid at normal ambient temperatures and is colorless.  Corn is the current primary grain source.  It has many of the same characteristics as methanol.  Currently ethanol is used primarily currently in a mixture with gasoline, usually no more than ten percent (10%) ethanol.

(1)	Advantages:

(a)	Ethanol emissions are quite low in particulates and NOx.

(b)	Like methanol, ethanol readily mixes with gasoline.

(c)	Ethanol is biodegradable and readily assimilates with water.

(d)	Ethanol is less corrosive and less toxic than methanol.

(e)	Ethanol is a domestically produced energy source.

(2)	Disadvantages:

(a)	The production process is extensive and the steps involved, i.e., planting, fertilizing, harvesting, shipping, processing, consume nearly as much energy as is created.

(b)	The energy output of ethanol, though higher than methanol, is still only about half that of diesel fuel, thus the range of ethanol powered vehicles is limited for a given fuel storage capacity.

(c)	Ethanol emissions have some visible smoke.

(d)	Ethanol produces formaldehyde; however, this can be offset with an exhaust after-treatment.

(e)	The distribution system and infrastructure for ethanol fueling are considerably less widespread than for gasoline and diesel.

c.	Clean diesel

Clean diesel was one of the alternative fuels approved in the Clean Air Act Amendments of 1990.  The first step being undertaken is further refining to reduce sulfur content and hence the significant particulate emissions caused by the sulfur.  Further steps are being developed and to add cetane boosters which increase efficient combustion.  Additives are also being developed to reduce aromatic hydrocarbons in the exhaust.

(1)	Advantages:

(a)	The additional processing costs are small, so clean diesel is cost effective.

(b)	All existing diesel engines currently in service can use clean diesel without modification.

(c)	The present systems for distribution of diesel fuel are unchanged and are fully usable with clean diesel.

(d)	Clean diesel retains the low level of diesel fuel volatility.  		This makes it safer than many of the other alternatives.

(e)	Clean diesel has a higher BTU value per gallon or equivalent gallon than any other alternative fuel, and thus provides more engine efficiency as well as more vehicle range.

(2)	Disadvantages:

(a)	Clean diesel is still relatively high in particulates and NOx.

(b)	Clean diesel is a fossil fuel and, as such, leaves us still dependent on foreign sources.

(c)	When operating under cold conditions, starting is a problem, as with all diesel fuels.

d.	Reformulated gasoline

Reformulated Gasoline is a specially blended fuel with the following properties: a lower vapor pressure that reduces evaporation during operation and refueling; more efficient combustion through the addition of high-octane oxygenates.  Reformulated gasoline aromatic levels have been lowered, which provides less in the way of hydrocarbon tail pipe emissions.

(1)	Advantages:

(a)	Reformulated gasoline is compatible with all existing gasoline engines.

(b)	The existing fuel-delivery infrastructure is unchanged by this change in fuel properties.

(c)	Reformulated gasoline is a cost-effective alternative in spite of some additional refining costs.

(2)	Disadvantages:

(a)	Currently there is insufficient oxygenate production and storage (as well as transportation) to provide the oxygenate when and where it is needed.

(b)	Like regular gasoline, reformulated gasoline has a lower caloric (BTU) value than diesel and, thus, provides less engine efficiency than diesel and less range for a given fuel capacity.

(c)	Reformulated gasoline is a fossil fuel and, as such, leaves us still dependent on foreign sources.

(d)	Present technology and federal emissions and energy standards will allow reformulated gasoline to be viable to the year 2000.  Significant improvements must take place if gasoline is to be used after that time, assuming present planned regulations remain in place.

2.	Gaseous alternative fuels

a.	Natural gas

Natural gas is primarily methane as it comes from the well, and it burns quite cleanly in its unprocessed state.  Natural gas has a higher ignition point (temperature) and a narrower fuel/oxygen mixture combustion range than other fuels.  Energy is consumed in processing natural gas to achieve sufficient vehicle storage (i.e., compression or cryogenic processes).  See Compressed Natural Gas and Liquid Natural Gas below.

b.	Compressed natural gas (CNG)

Compressed Natural Gas, or CNG, consists primarily of mixtures of hydrocarbon gases and vapors, consisting principally of methane (CH4) in gaseous form, which is compressed for use as a vehicular fuel.

(1)	Advantages:

(a)	Natural gas is readily available as a domestic energy source, is inexpensive, and has generally lower emissions than most other alternative fuels.

(b)	CNG already is in use as a viable alternative for light-duty vehicles.  The American Gas Association reports over 700,000 natural gas vehicles in operation in 38 countries.

(c)	The cleaner burning minimizes carbon buildup thus, increasing oil change intervals and reducing maintenance.

- Disadvantages:

- The pressure of CNG requires heavy storage tanks.  The     tanks are large even for short-range use.  Those two          factors reduce cargo capacity.  Maintaining reasonable        cargo capacity restricts tank size and limits range.              Lower caloric (BTU) value per equivalent gallon than          diesel also limits engine efficiency and vehicle range.

- The high pressure which the CNG fuel storage system        must endure requires careful design and location on the     vehicle, protection from damage, plus periodic                   maintenance and upkeep.  Periodic tank testing for            structural safety is required and replacement during the      life cycle of the vehicle may be necessary.

(c)	Refueling time is dependent on the type of fueling system used, and can be quite lengthy.   There are two methods: “slow-fill” which takes from 5 to 8 hours and is typically called “overnight” or “time-fill” refueling and “fast-fill” which takes about 5 to 10 minutes and requires high-volume compression and special filling apparatus.

(d)	Natural gas compression and refueling equipment is expensive and must be maintained.  Fast-fill capability requires an additional “cascade” of high volume storage cylinders, which adds considerable expense to the fueling station.

(e)	There are composition variations in natural gas and percentage of methane content from one area to another. Additional processing is required to get uniform natural gas available in all areas.

(f)	Natural gas has poor lubricative properties.

c.	Liquid natural gas (LNG)

Liquid natural gas utilizes the same natural gas source (primarily methane) as CNG, but requires purification of the gas and cooling and storage below -260 degrees Fahrenheit to liquefy the natural gas.  Converting natural gas to liquid form provides storage of a much greater amount on the vehicle than can be achieved in the gaseous state.

(1)	Advantages:

(a)	Liquid natural gas has all of the combustion advantages of compressed natural gas, is readily available, clean burning and generally produces lower emissions than alternatives other than CNG.

(b)	An engine will operate just as easily on LNG as it does on CNG.  Though one is stored by compression and the other by cryogenics, when either gets to the point of combustion, it is natural gas.

(c)	The range of an LNG is greater than that of CNG due to the fuel density.

(d)	The LNG fuel system pressure is less than one hundred psig (100) as compared to three thousand psig (3,000) in a CNG system.

(e)	LNG provides almost pure methane with known performance characteristics.

(2)	Disadvantages:

(a)	Maintaining the super-cool temperature requires large, heavy, highly insulated tanks which still forces compromise between vehicle range and cargo carried.

(b)	Equipment to super-cool and liquefy gas is expensive to purchase, operate and maintain.

(c)	Liquid natural gas can be kept in the insulated storage tank for 7 to 10 days.  After that, it must be bled off maintain the cold temperature required to hold the gas in liquid form.

(d)	The bleeding-off process releases hydrocarbons which, in turn, requires treatment to avoid direct release into the atmosphere.

  - Natural gas has poor lubricative properties.

d.	Propane (also known as LIQUEFIED PETROLEUM GAS OR LPG)

Propane or LPG, is sometimes available directly from wells, but is normally produced as a by-product of the gasoline refining process.  It has been used for a number of years in light-duty commercial vehicles in urban areas around the world.

(1)	Advantages:

(a)	Propane burns relatively clean.  It emits less NOx and contains less particulate matter than diesel; and emits less carbon monoxide and fewer hydrocarbons than gasoline.

(b)	The cleaner burning maximizes carbon build-up in the engine and hence resulting in less maintenance.

(c)	Propane starts better in cold weather than either diesel or gasoline.

(d)	The infrastructure for distribution and storage of propane is relatively widespread.

(2)	Disadvantages:

(a)	As with CNG, propane requires large and heavy fuel tanks to achieve reasonable driving range, due to reduced engine efficiency per equivalent gallon.

(b)	Propane requires the use of relatively low compression ratios hence, has lower economy.

(c)	Propane vapors, like gasoline, are heavier than air and volatile.  These explosive mixtures settle in service pits or other spots; therefore, indoor storage can be a safety concern.

(d)	As a by-product, propane, is dependent on the gasoline process which limits supply.  Further, it does little toward the reduction of dependency of foreign oil.

(e)	Propane has poor lubricative properties

3.	Electric power

The use of electricity as a power source for school buses is an emerging technology that is under considerable research due to the potential for reduced overall emissions.   Research is centering on ways to increase the capacity and reduce the weight of batteries, as well as improving the motors used to power the vehicles and the associated electronics.  Recharging technology is also developing rapidly.  Most of these efforts have the goals of improving the range and performance of electric vehicles, reducing their costs, and addressing operational concerns, such as recharging.

(1)	Advantages:

(a)	Electric power vehicles produce no tail pipe emissions.

(b)	The electricity distribution system is currently available; power lines are already in place.

(c)	Electricity can be, and often is, produced from renewable, domestic energy sources.

(d)	Electric power vehicles are extremely quiet, due to the lack of internal combustion engines.

(e)	Electric school buses can be produced as hybrid vehicles, which would have a small internal combustion engine to recharge batteries, or to supply heating systems or various other chassis accessories.

(f)	The cost per mile to operate electric power vehicles is low; power source maintenance is practically nil, compared to internal combustion engines.

(2)	Disadvantages:

- Electric power vehicles have low range, due to battery weight and limited electrical storage capacity of current batteries.

(b)	Electric power vehicles may not eliminate overall emissions and/or foreign oil dependency if electricity to charge vehicle batteries is produced from coal or oil.

(c)	Current cost of electric power systems for vehicles, including batteries, is extremely high.

(d)	Battery disposal is an environmental concern.

(e)	Significant weight of current batteries limits passenger carrying capacity.